# Name: __________	Class: ___	Date: ________

# Solution sheet task 1:

# Weather station

Tasks 1 and 2 turn the sensor station into a local weather station. In the experimental tasks, the weather station is connected to a web server via MQTT that displays the measured data and transmitted images in a dashboard.

## Construction task

See building instructions.

## Programming tasks

Configuring the sensors and actuators:

1. Measuring humidity, temperature, and air pressure

1a. Display configuration (example):

Configuring the display output

1b. Program (example):

IoT_Meteorological_Station.ft

2. Barometer

2a. Expanding the display configuration (example):

Expanding the display output

2b: Program (example):

The temperature values must be converted into Kelvin before the calculation: 0°C = 273.15 K.

IoT_Barometer.ft

## Experimental tasks

1. Determining temperature using the NTC resistor

1a. The resistance of the thermistor and the associated measured temperature values for the environmental sensor can easily be determined via a Log output on the console (see example program below).

Three measured values can be used to determine the coefficients a, b, and c of the Steinhart-Hart equation [4, 5] (T: Temperature in Kelvins, R: NTC resistance in Ohms):

1T=a+b∙lnR+c∙ln⁡(R)3

| Resistance value | Temperature |
| --- | --- |
| 1500 Ω | 25.0 °C |
| 2362 Ω | 13.7 °C |
| 2530 Ω | 11.8 °C |

Example measurement

The measured values are dependent on the NTC resistance and may deviate.

Coefficients: a = 0.004535418128, b = -0.0003767491105, c = 0.000004023802790

1b. Expanding the display:

Configuring the display output of NTC temperature measurement

1c. Expanding the program (example):

Calculating the temperature value based on the NTC resistance and output of the measured values for the NTC resistor and temperature sensor on the console.

Here as well, the temperature value calculated from the thermistor must then be converted into Kelvins: 0°C = 273.15 K.

IoT_Barometer_with_NTC_resistor.ft

2. Data visualisation on an IoT server

2a. Configuring the dashboard on the IoT server in the fischertechnik cloud:

Configuring the display in the dashboard

2b. Expanding the program (example):

IoT_MQTT_Barometer.ft

3. Webcam

3a. Expanding the dashboard in the fischertechnik cloud:

Dashboard with webcam

3b. Expanding the program (example):

IoT_MQTT_Barometer_with_Webcam.ft

Annex

# Task 1: Weather station

## Required materials

PC for program development, local or via web interface.

USB cable or BLE or WiFi connection for transmitting the program to the TXT4.0.

Example programs “IoT_MQTT.ft” and “IoT_Webcam.ft”

Account in the fischertechnik cloud

## Further information

[1]	Wikipedia: Barometric formula.

[2]	Online diagram editor for creating state transition diagrams (drawio format): https://www.diagrammeditor.de/

[3]	fischertechnik: NTC resistance. Data sheet, Art. no. 36437.

[4]	Stanford Research Systems (SRS): Thermistor Calculator. V1.1

[5]	Dirk Fox: “Einmessen” eines digitalen Messgeräts (“Calibrating” a digital measurement device). ft:pedia 1/2013, p. 39-48.