# Solution sheet Gears model 1 – Beam balance (lever)

Students are supported in constructing and solving some of the tasks with provided building instructions (see attachment). This is indicated at the start of the solution sheet in tasks where appropriate.

Students receive a copy of the measurement scales to cut out.

## Topic task

The beam balance can be calibrated with any “unit weight”. The markings on the scale should be divided into suitable graduation marks.

1. The distance between the graduation marks must be uniform, since the counterweight is pushed in a linear direction to increase the weight in the scale pan. The distance between two 10 g markings should be about 5 cm.

2. The scale can be used to weigh a maximum of a little more than 35 g.

3. When the scale is in balance, meaning that the beam balance is aligned exactly horizontally and the scale indicator points to the tip of the red angle disc, then the torques on the right and left sides of the axle are equal.

## Experimental task

1. The measurement range of the scale can be doubled by doubling the length of the right side of the beam balance on the image, or halving the length of the left side. This will double the movable counterweight on the right side of the beam balance.

2. You can achieve a finer resolution for the scale by reducing the counterweight or extending the left side of the beam balance. To retain the measurement range, you must extend the right side of the beam balance accordingly.

Enclosure

Model 1: Building instructions for beam balance, sheet with empty scales to cut out