# Gears model 1 – Beam balance (lever)

To determine the weight of an object, we must know the ratio of the weight to a fixed reference or basic value. The scales shown in this and the next tasks can be used to bring two movable levers into balance. The transformation to the reference weight is then carried out using a suitable scale.

## Construction task

Fig. 1 Beam balance

Build the beam balance shown in Fig. 1. It works according to the lever principle: If you increase the weight on the scale pan (in the left image), you must push the movable weight from the four yellow basic building blocks (in the right image) on the beam outward to balance it out so that the indicator (the black axle) points exactly to the tip of the red angle disc.

Cut out the template for the scale and fasten it to the black angle brackets with two S-latches on the right and left. Take a black pen and mark the “zero position” of the weight with a line and a “0” (with the scale pan empty), which is the point at which the tip of the red angle disc under the weight on the scale points.

## Topic task

Now the scale still needs to be calibrated. To do so, you must add “unit weights” to the scale pan. If you do not have exact weights available, you can also use basic building blocks as the “unit weight”. (They weigh a little over 5 g.)

Mark the position of the red angle disc under the weight on the scale with a line when the weight is set so that the pointer on the scale points exactly to the lower angle disc. You can complete the scale in this way. Check your calibration to ensure it is correct with objects of different weights whose weights are known to you.

1. What is the distance between two 10 g markings on the scale?

2. What is the maximum weight you can measure using the scale?

3. When the scale is balanced, what physical variables are equal?

## Experimental task

1. How can you change the scale so that the measurement range is doubled? Name at least two possibilities.

2. Imagine you want a more precise resolution for the scale. How can you achieve this? (There are also multiple possibilities for this as well.)