# Gears model 2 – Letter scale (lever)

The following type of mechanical scale would more accurately be called an “inclination balance” and is still used today very broadly as a letter scale.

## Construction task

Fig. 1 Letter scale

Build the letter scale shown in Fig. 1. In contrast to a beam balance, it is not necessary to adjust any weights with this scale: The weight placed on the (right) scale pan deflects the left pointer (yellow base module with angle disc on the tip). The scale is in a stable balance.

Cut out the template for the letter scale and fasten it to the yellow, curved angle brackets with two S-latches on the right and left. Take a black pen and mark the “zero position” of the scale with a line and a “0” (with the scale pan empty), which is the point at which the tip of the red angle disc on the scale points.

## Topic question

Now this scale still needs to be calibrated. To do so, you must add “unit weights” to the scale pan (see task 1). Make lines to mark the position of the tip of the red angle disc on the scale. Check your calibration to ensure it is correct with objects of different weights whose weights are known to you.

What is the maximum weight you can measure with this scale?

## Experimental task

1. How can you increase the measurement range for this scale?

2. How can you refine the measurement for this scale? (Here as well, there are multiple ways to achieve this.)

3. Why are the distances between two 10 g markings, for instance, on the scale smaller as the weight increases?