# Solution sheet Gears Model 3 – Pulley and wheel and axle

Students are supported in constructing and solving some of the tasks with provided building instructions (see attachment). This is indicated at the start of the solution sheet in tasks where appropriate.

Note on technological history: When the pulley was invented is not known. The oldest known descriptions of pulleys come from the Roman author Marcus Vitruvius Pollo (approx. 80-15 BC), who described the engineering knowledge of the day in his “Ten books about architecture”. These books included the wheel and axle, which was used in Roman building cranes.

## Topic question

1. The exact number of crank revolutions depends on the thickness of the rope drum, or the amount of rope that has already been wound. Without a pulley, this will be a full three rotations, in case a) around six and a half, in case b) around 10 and in case c) 13.

2. The force amplification is inversely proportional to the required rope length. Case a) doubles the force, case b) triples it and case c) quadruples it. The force amplification can be counted based on the number of rope loops.

3. This is called a “factor”: The number of rollers used is the factor by which force is amplified.

## Experimental task

1. The force amplification of the wheel and axle corresponds to the ratio between the long lever and the radius of the rope drum. If the rope is wound directly onto the drum (diameter 0.7 cm), and if a hand grips the centre of the snap-on adapter, then the amplification in case a) is 4.5/0.35 ≈ 12.85 and in case b) 6/0.35 ≈ 17.14.

As the quantity of rope wound increases, the radius of the rope drum also increases, and the force amplification is reduced. However, when comparing the wheel and axle with the crank, it should be noted that the crank itself amplifies force by around a factor of 1.2/0.35 ≈ 3.42.

2. The path length that must be completed when turning the wheel and axle increases proportional to the force amplification. (It is easy to check the plausibility of this, since the circumference is U = 2 π r; multiplying the radius by the same factor is used to calculate the circumference.)

3. It is relatively easy to generate a high force amplification on a wheel and axle. The pulley requires a large number of additional rollers to do the same thing. If the rollers of the pulley are arranged one on top of the other, the stroke length is shortened. However, this can be avoided by arranging the rollers in parallel. The additional rope length required must, however, fit on the rope drum. The advantage of this is that the weight to be lifted is distributed over the rope slings; a more stable tow rope is not required for a higher weight.

The lever on the wheel and axle and tow rope, in contrast, must absorb the entire increased force or the weight to be lifted, and must be stronger in order to handle the greater weight. A longer lever on the wheel and axle also requires a larger, round running surface around the wheel and axle. Since the tensile force on the rope drum increases with the increase in weight lifted, a wheel and axle should also have a pawl.

Enclosure

## Building instructions and templates for the gears and models:

Model 3: Building instructions pulley with cable winch, building instructions pulley with wheel and axle