# Solution sheet Gears Model 5 – Cardan shaft

Students are supported in constructing and solving some of the tasks with provided building instructions (see attachment). This is indicated at the start of the solution sheet in tasks where appropriate.

Note on technological history: The name “cardan joint” comes from the Italian scholar and mathematician Gerolamo Cardano (1501-1576), who described what would later be called “cardan suspension” in 1550 in “De Subtilitate”. Actually, this type of suspension, also called a “gimbal suspension” had already been described as early as 230 BC by Philo of Byzantium, and was drawn many times by Leonardo da Vinci (1452-1519) around 1500. There is no known publication by Cardano of a universal or cardan joint based on the cardan suspension. The oldest known description of the cardan joint comes from Caspar Schott (1608-1666) in his book “Technica Curiosa” from 1664.

The correction of the cardan error in a cardan shaft was discovered by English physicist Robert Hooke (1635-1703) in 1683.

Students receive a copy of the protractors to cut out.

## Construction task

The input and output axles meet at an angle of about 119°.

## Topic task

| Rotational angle input | Rotational angle output | Δ | Rotational angle input | Rotational angle output | Δ |
| --- | --- | --- | --- | --- | --- |
| 0° | 0° | 0° | 90° | 90° | 0° |
| 15° | 17° | 2° | 105° | 95° | -10° |
| 30° | 42.5° | 12.5° | 120° | 102.5° | -17.5° |
| 45° | 57.5° | 12.5° | 135° | 115° | -20° |
| 60° | 70° | 10° | 150° | 125° | -25° |
| 75° | 77.5° | 2.5° | 165° | 150° | -15° |
| 90° | 90° | 0° | 180° | 180° | 0° |

## Experimental task

1. The maximum adjustable angle of the cardan joint (smallest angle between the input and centre portion of the cardan shaft) is approx. 117°.

3. Alternative gearing mechanisms include, for instance, a spur gear drive (two toothed gears of the same size on the input and output axle, with any toothed gear between them so that there is no reversal of direction), a chain or belt drive. The disadvantage of a spur gear drive is a loss in efficiency (up to 10%); the disadvantage of a belt drive is the force-locking connection. Chain drives have a significantly lower maximum speed than a spur gear drive (factor of 10 to 30).

Enclosure

## Building instructions and templates for the gears and models:

Model 5: Cardan joint building instructions, page with protractors for the cardan joint to cut out, cardan joint building instructions with protractors