# Tasks Gears Model 5 – Cardan shaft

Sometimes, the input and output shafts of a gear drive are neither in line nor parallel with one another; instead, they intersect with one another at an obtuse angle. Then the direction of motion of the shaft must be changed. This is done using a cardan drive, also called a universal joint or cardan joint.

## Construction task

Build the cardan joint shown in Fig. 1. What is the angle between the input and output axles?

Fig. 1 Cardan joint

If you drive the input shaft using the crank, you will see that the output shaft does not move in the same even manner. It sometimes turns faster and sometimes slower than the input shaft. This effect is called “cardan error”.

## Topic task

Now, extend the input shaft with a snap-on coupling and a snap-on axle 45, and the output shaft by replacing the snap-on axle 30 with a snap-on axle 45. Add a hub 60 to the input shaft with a flat hub so that the hub nut points towards the cardan joint, and add a second bearing before inserting the snap-on crank once again. Replace the wheel with a second hub 60 with flat hub; the hub nut in this case must point away from the cardan joint. Finally, extend the bearing of the input shaft and output shaft with a module 15 with attached 60° angle module, to be able to better read the angle disc (see Fig. 2).

Fig. 2 Adding two protractors to the cardan shaft.

Cut out the two angle discs (Fig. 3), cut a hole in the middle and push them onto the two axles in front of the hubs so that they are clamped on the input axle between the crank and hub and on the output axle between the snap-on coupling and hub. You can also attach them to the hub using transparent tape.

Fig. 3 Disc with graduation as protractor

Bring the cardan shaft to the same position as shown in Fig. 2 and align the two protractors so that the tips of the red angle disc are “flush” with the 0° indicator. Now, turn the crank in 15° steps from 0° to 180° and note the angle values indicated on the second protractor on the output axle.

Enter the measurement results and respective deviation between the input and output axles (“delta”) into the following table.

| Rotational angle input | Rotational angle output | Δ | Rotational angle input | Rotational angle output | Δ |
| --- | --- | --- | --- | --- | --- |
| 0° | 0° | 0° | 90° | 90° | 0° |
| 15° |  |  | 105° |  |  |
| 30° |  |  | 120° |  |  |
| 45° |  |  | 135° |  |  |
| 60° |  |  | 150° |  |  |
| 75° |  |  | 165° |  |  |
| 90° | 90° | 0° | 180° | 180° | 0° |

## Experimental task

As you have seen, the cardan error of a cardan shaft can be significant. The greater the angle by which the shaft is deflected, the greater the error.

However, there is one interesting point to note: If we “connect” two cardan joints to a cardan shaft one after the other so that the input and output shafts are parallel, then the cardan errors of the two cardan joints cancel one another out. Therefore, cardan joints in even drives are generally only used with a low deflection, or used in pairs when designing a cardan shaft.

Fig. 4 Cardan shaft with two cardan joints

Fig. 4 shows one such cardan shaft. Build this shaft, and use it to solve the following tasks.

1. What is the maximum angle at which the two cardan joints still turn “cleanly”?

2. Mount the two protractors on the input and output shafts of the cardan shaft, and check to see whether the cardan error has really been eliminated.

3. What other gearing mechanisms can you think of that can achieve the kind of offset between input and output that we see with a cardan shaft? What are their advantages and disadvantages, compared to a cardan shaft?