# Tasks Gears Model 7 – Clock gears

## Construction task

One of the most refined technical details of a clock is that multiple hands – at least the hour and minute hands – speed on the same axle at different speeds.

This is achieved using a “freewheel hub”: The minute hand is driven by the metal axle in Fig. 1, while hub 60 placed loose on the same axle with the black freewheel hub is moved by the Z40 behind it.

Fig. 1 Clock gears – Base construction

The clock should be operated with a single input shaft: the axle on which the minute hand sits. If the hour hand (hub 60 or Z40) should turn depending on this shaft: What gear ratio would then be required between the minute hand and hub 60?

## Topic task

Now, build the gear to drive the hour hand based on the hand shaft as the input shaft.

To do so, proceed “backwards”: Look at the Z10 below the Z40. What is the gear ratio here between the shaft of the Z10 and hub 60? What gear ratio is required between the shaft of the Z10 and that of the minute hand?

Now, ask yourself what toothed gears you could use to create this gear ratio, then build the gear ratio so that it connects the minute pointer shaft with the Z10 under hub 60.

## Experimental task

1. Clock faces normally show 12 hours, but this is not always the case. It would be more natural to show all 24 hours of the day in one rotation, dividing the clock face into 24 hours. What would a gear used for this purpose look like? Can you build one?

2. We are still missing an input shaft for the clock. We want to create one using a crank.

Build a crank drive to the side of the clock with a toothed gear with catch, so that each time the catch “clicks” (on each tooth of the toothed gear), the clock hands move exactly one minute forward.