# Solution sheet Gears Model 8 – Planetary gear

Students are supported in constructing and solving some of the tasks with provided building instructions (see attachment). This is indicated at the start of the solution sheet in tasks where appropriate.

Note on technological history: In 1780, James Pickard received a patent for a crankshaft drive, although such drives has already been in use for at least 1500 years. He attempted to use it to blackmail James Watt (1736-1819), who was close to completing his first “steam engine”. Afterwards, Watt's congenial assistant William Murdoch (1754-1839) quickly invented an epicyclic gear consisting of two coupled toothed gears, of which one revolves like a “planet” around the other (the “sun”), in order to circumvent Pickard’s patent. Watt then received his own patent for this alongside his expansion steam engine in 1781 (patent no. GB 1321).

Students receive the building instructions for the planetary gear with fixed carrier.

## Construction task

The coaxial bevel gear reverses the direction of rotation.

The coaxial crown wheel results in a gearing down of 1:3.2.

## Topic task

The two gearing mechanisms are identical, since both the internal and external gears have 30 teeth. In the planetary gearing mechanism, the two planetary gears reverse the direction (the rotational direction remains the same for the internal gear). In a simple spur gear, the transition from Z10 to Z30 results in a reversal of direction.

The planetary gear with fixed carrier and sun gear on the input shaft, therefore, results in a gearing ratio of 1:3, gearing down with a change of direction.

## Experimental task

1. The fischertechnik Z30 internal gear can be used to construct the following two other planetary gears: a) planetary gear with fixed ring gear:

Interesting variant of this gearing mechanism: The shaft of a planetary gear as output is provided by a stirrer:

b) Planetary gear with fixed sun gear:

2. These fischertechnik planetary gears can be used to implement the following gearing ratios:

| Fixed | Input | Output | Gearing ratio | Change in direction |
| --- | --- | --- | --- | --- |
| Carrier | Sun gear | Ring gear | -3 | yes |
| Carrier | Ring gear | Sun gear | -0.33 | yes |
| Ring gear | Sun gear | Carrier | 4 | no |
| Ring gear | Carrier | Sun gear | 0.25 | no |
| Sun gear | Carrier | Ring gear | 0.75 | no |
| Sun gear | Ring gear | Carrier | 1.33 | no |

3. You can achieve the greatest gearing down by coupling the first (in the table) and third gears. This results in a gearing ratio (with change in direction) of -12.

Enclosure

## Building instructions and templates for the gears and models:

Model 8: Building instructions for planetary gear with fixed carrier, building instructions for planetary gear with fixed ring gear, building instructions for planetary gear with fixed sun gear.