# Tasks Gears Model 8 – Planetary gear

Gears in which the change in motion is coaxial are particularly well-suited for many practical applications. In these gears, the input and output shafts are in line with one another. They are compact, easy to construct, and simple to combine with one another.

## Construction task

Fig. 1 shows a coaxial gear mechanism with bevel gears. Build this gear mechanism. What change in motion does it generate?

Fig. 1 Coaxial bevel gear

A coaxial gear can also have a gearing ratio. The gearing mechanism in Fig. 2 uses a crown wheel. Build this mechanism. What is the gearing ratio?

Fig. 2 Coaxial step-up gear unit with crown wheel

## Topic task

Planetary gears are a special type of coaxial step-up gear units. They are generally constructed as spur gears, meaning that the teeth of the sprockets are vertical to the axle (shaft). Planetary gears consist of

a “sun gear” (a toothed gear in the centre).

multiple “planetary gears” that “revolve” around the sun gear, with shafts connected to one another via a carrier, and

a “ring gear” into the internal teeth of which the teeth of the planetary gears interlock.

Planetary gears can be constructed to be very compact. Depending on which of the three shafts of the planetary gear – the shaft of the sun gear, that of the carrier or that of the ring gear – is fixed, the gearing ratio will change.

First, observe and build the following planetary gear with fixed carrier and the sun gear on the drive shaft (Fig. 3):

Fig. 3 Planetary gear with fixed planetary gear carrier and sun gear input shaft

The gearing ratio of the planetary gear is identical to that of the following simple spur gear (Fig. 4):

Fig. 4 Spur gear equivalent to the planetary gear

Why is this? Please explain. What is the gearing ratio of the planetary gear with fixed carrier and sun gear as the input?

## Experimental task

1. The carrier in the planetary gear in Fig. 3 is fixed. Build another planetary guild where either the sun gear or the ring gear is fixed.

2. What gearing ratios can be achieved with fischertechnik spur planetary gears with internal gear Z30? Complete the following table:

| Fixed | Input | Output | Gearing ratio | Change in direction |
| --- | --- | --- | --- | --- |
| Carrier | Sun gear | Ring gear |  | yes/no |
| Carrier | Ring gear | Sun gear |  | yes/no |
| Ring gear | Sun gear | Carrier |  | yes/no |
| Ring gear | Carrier | Sun gear |  | yes/no |
| Sun gear | Carrier | Ring gear |  | yes/no |
| Sun gear | Ring gear | Carrier |  | yes/no |

As you have seen, some of these gearing mechanisms reverse the direction. We use a minus symbol (“-”) to designate these in the gearing ratio equation.

3. Connecting multiple planetary gears in a series can be used to create high gearing ratios. Observe the three different planetary gears. Which two (different) gears would you connect to implement the greatest gearing down?