# Name: __________	Class: ___	Date: ________

# Solution sheet model 10

# Ball passing 4 – Linear feed

## Construction task

## Topic tasks

1.	The structural strut 45 acts as a spring here. It is “wound up” by the rotating link 15 and suddenly released. This makes the S-rivet at the end of the strut strike the table tennis ball from below and make it shoot upwards. If everything has been adjusted properly, the ball is launched over the hill without leaving the track.

2.	The models suggested can and should be adapted or supplemented by new structures. One prerequisite is that attention is paid to the so-called interfaces of new modules, so that they can be connected together.

## Experimental tasks

1.	One revolution of the crank leaks to one upward and downward movement of the lift. This is why a table tennis ball can be lifted with one turn of the crank.

2.	If the crank is turned evenly, the upward and downward movement is not at the same speed.

The movement is always slower when the second axle inserted in gear wheel Z30 is at the front (towards the entrance of the model). It is faster when the Z30 axle is at the rear (towards the exit of the model). The axle in the Z30 is then closer to the axis of rotation of the strut, which is acting as a lever. The lever thus translates the movement of the axle in the Z30 into a faster movement at the end of the lever (when the lift is to be raised).

Depending which direction the crank is turned in, this can be the upward or downward movement.

So the following applies for this model:

a)	If the crank is turned clockwise (to the right), the upward movement is slower and the downward movement is faster.

b)	If the crank is turned anti-clockwise, it moves up faster (and a bit harder) and down slower.

Attachments

# Ball passing - Linear feed

## Supplementary material

- Table tennis balls

## Further information

[1]	Wikipedia: Slideway.