# Name: __________	Class: ___	Date: ________

# Solution sheet model 4

# Beam balance

## Topic tasks

1.	The beam of the balance tilts to the side. The heavier part is further down. The pan with the lighter weight is raised.

2.	If the parts in the two pans weigh the same, the beam the balance aligns horizontally

3.	The structural rivet and the fixed building block 7.5 serve to be able to read off exactly when the balance is in “equilibrium”, in other words when the same load acts on both sides.

4.	We can use the balance beam to check of two weights weigh the same. The object to be weighed is placed in one of the pans and the other pan is filled with known weights until the balance is in equilibrium.

The weight determined is the sum of all the known weights placed in the second pan. However, the beam balance does not have a scale where we can read off the weight of an object directly

## Experimental tasks

1.	If both pans contain the same objects, but these are at different distances from the middle of the beam:

a)	the beam balance no longer reliably reaches equilibrium although both objects weigh the same.

b)	The objects must be placed at the same distance from the middle of the beam. The further out an object is placed, the greater the effect on the leverage acting on the balance.

2.	If one of the pans on the balance cannot move freely, this has a similar effect as in experimental task 1: The pans are no longer at the same distance from the middle of the scale when the scale is deflected. So the result can be falsified.

Attachments

# Beam balance

## Further information

[1]	Wikipedia: Beam balance.