# Name: __________	Class: ___	Date: ________

# Solution sheet model 6

# Crane

## Construction tasks

1.	Without tower and pawl:

2.	With pawl:

3.	With pulley:

4.	With tower:

## Topic tasks

1.	Without pawl, pulley and tower:

a)	The crane hook can be pulled down easily. It could hardly bear any weight because nothing is holding the rope tight.

b)	If you look carefully, you can see that the Z10 is turning faster than the Z20. Every time the Z20 revolves once, the Z10 (and the crank) revolve twice. This means the crane rope is wound up more slowly than without the gear drive - but you don’t need as much force, and can still pull the rope with more force.

c)	The flatter the crane arm angle, the further away the material to be lifted can be from the crane.

The steeper the crane arm, the closer the material to be lifted has to be to the crane. However, it can be lifted higher because the crane arm is now raised further.

2.	The pawl solves the problem from task 1. a). If it is resting on Z10, the rope can no longer unwind itself (providing you have made sure the rope comes from the rope drum “from the bottom”). It is only now possible for the crane to hold a weight without the crank having to be held tight.

To lower the crane rope, you just have to lift the pawl a little so that the teeth of the gear wheel Z10 can rotate freely under it.

3.	Like the gear drive, the pulley ensures that the crane hook is raised even more slowly when you turn the crank. But in return, the weight suspended from the hook is now distributed to two rope strands. Each of these only has to bear half the weight.

Since we are only pulling one of the rope strands with the crank and rope drum, we only need half the force for this as well. This means we can lift larger weights using less effort! The only thing is, lifting is now slower.

4.	With tower:

a)	If the weight on the crane hook is too heavy, the tower will topple over. In the case of a real crane, this would be a dangerous accident! (This is true without the tower as well - give it a try!)

b)	If the crane is at a flatter angle, but projects out further, the crane will tilt with even smaller weights than it if its at a steeper angle and ends close to the base.

## Experimental tasks

1.	Longer bases provide further protection against the crane toppling over if weights are too heavy. In other words, larger weights can be attached. However, you have to be careful when slewing the crane if the bases are not the same length in all directions.

2.	If the bases are lengthened to the “rear”, it makes no difference to the lifting of heavy weights at the front but helps prevent tilting if the crane is slewed to the rear.

Attachments

# Crane

## Supplementary material

- A household rubber band or other string can be useful to attach objects to the crane hook.

- Any kind of objects – fischertechnik parts, pens, whatever - can be lifted using the crane.

## Further information

[1]	Wikipedia: Crane. (This article presents other types of crane as well.)

[2]	Wikipedia: Ratchet.