# Name: __________	Class: ___	Date: ________

# Solution sheet model 7

# Ball passing 1 - Rotary lever

## Construction task

## Topic tasks

- The levers pick up the ball from below and lift it. The red flat building blocks on the end prevent the ball rolling off the lever. The ball is guided by the track the whole time, which stops it rolling out to the left or right.

Once it gets to the top, the ball rolls down the track exit. If you crank “the wrong way round”, the balls are not lifted - the model only works in one direction.

2.	The models suggested can and should be adapted or supplemented by new structures. One prerequisite is that attention is paid to the so-called interfaces of new modules, so that they can be connected together.

## Experimental tasks

1.	You have to turn the crank twice for the levers to rotate once. This is due to the “step-down ratio” of the black gear wheel Z10 to the red gear wheel Z20 (which we learned about on the “Crane” sheet). When Z10 has been turned once by the crank, wheel Z20 has only been turned by 10 teeth. This is why you need two revolutions of Z10 for Z20 to make one full revolution.

2.	Our model has two lever arms. This means that two balls can be lifted per revolution. Although the levers only revolve at half the speed the crank is turned at, this results in one ball lifted per revolution of the crank.

Attachments

# Ball passing - Rotary lever

## Supplementary material

- Table tennis balls

## Further information

[1]	Wikipedia: Gear