# Name: __________	Class: ___	Date: ________

# Solution sheet model 8

# Ball passing 2 – Four-bar linkage

## Construction task

## Topic tasks

1.	a) Eccentric gear

The axle with the I-strut 45 inserted in the gear wheel turns

[x] outside the centre of the gear wheel

[  ] inside the centre of the gear wheel

b)	Four-bar linkage

Find the four rotating joints, which are all connected to one another by lever: The centre of rotation of the gear wheel with the crank, the axle in the gear wheel with the rotating mounted strut (the “lever” here is the gear wheel itself), the rotating mount of the top of the strut in the long lever as well as the rotating support of the large lever at the model exit. Study the mechanisms and how they work in detail.

2.	The models suggested can and should be adapted or supplemented by new structures. One prerequisite is that attention is paid to the so-called interfaces of new modules, so that they can be connected together.

## Experimental tasks

1.	One revolution of the crank raises and lowers the large lever once. For this reason, one revolution can lift and transport one ball.

2.	The structural girder hanging downwards is important if several balls are inserted once after the other. It prevents a ball slipping under the lever when the lever is in the top position. Without it, the ball would roll under the lever and block it.

Attachments

# Ball passing - Four-bar linkage

## Supplementary material

- Table tennis balls

## Further information

[1]	Wikipedia: Eccentric.

[2]	Wikipedia: Four-bar linkage.

[3]	Wikipedia: Mechanical linkage.

[4]	Prof. Dr.-Ing. habil. Jürgen Dankert: Planar quadrilateral linkage.