# Name: __________	Class: ___	Date: ________

# Solution sheet model 9

# Ball passing 3 – Launcher

## Construction task

## Topic tasks

1.	The structural strut 45 acts as a spring here. It is “wound up” by the rotating link 15 and suddenly released. This makes the S-rivet at the end of the strut strike the table tennis ball from below and make it shoot upwards. If everything has been adjusted properly, the ball is launched over the hill without leaving the track.

2.	The models suggested can and should be adapted or supplemented by new structures. One prerequisite is that attention is paid to the so-called interfaces of new modules, so that they can be connected together.

## Experimental tasks

1.	You have to turn the crank twice for the levers to rotate once. This is due to the “step-down ratio” of the black gear wheel Z10 to the red gear wheel Z20 (which we learned about on the “Crane” sheet and in ball passing 1 - “Rotary lever”). When Z10 has been turned once by the crank, wheel Z20 has only been turned by 10 teeth. Since it has 20 teeth, two revolutions of Z10 are need for one complete revolution of Z20.

2.	As in ball passing 1 - “Rotary lever“, we have two lever arms, namely both ends of the link 15. One revolution of the lever can thus lift one ball with each lever arm - in other words two per complete revolution. So although the levers only revolve at half the speed the crank is turned at, this results in one ball lifted per revolution of the crank.

Attachments

# Ball passing - Launcher

## Supplementary material

- Table tennis balls

## Further information

[1]	Wikipedia: Leaf spring (in: Spring).