# Name: __________	Class: ___	Date: ________

# Model 10

# Ball passing 4 – Linear feed

Note: You need at least one table tennis ball. You can use several at the same time, however, especially when several ball passing modules are built up connected to one another.

## Construction task

Build the model ball passing 4 according to the building instructions. Observe the following points:

- The building block 30 must slide easily between the two structural struts, but be guided safely. For this, the strut supports must be approx. 1 mm apart at the base.

- The building block 30 should be placed between the struts in such a way that the groove on its underside is aligned along the model.

- The horizontal prone strut should engage in the lower groove of the building block 30. The strut must be above the axle with the clip inserted in gear wheel Z30:

## Topic tasks

1.	Place a table tennis ball at the entrance (the shorter part of the track) and turn the crank clockwise (to the right). Describe briefly how rotating the crank leads to the ball being lifted.

_______________________________________________________

_______________________________________________________

_______________________________________________________

2.	If several modules are built up at the same time, connect them to form a longer track! All you have to do is insert the pins of building block 15 at the end of the module into the grooves of building block 15 at the entrance to the next module.

## Experimental tasks

1.	How often do you have to turn the crank for one ball to be lifted?

_______________________________________________________

_______________________________________________________

_______________________________________________________

2.	Compare the speed of movement of the “lift”

a)	during upward and downward movement and

b)	if you change the direction of rotation of the crank.

Can you see what happens and why?

_______________________________________________________

_______________________________________________________

_______________________________________________________

_______________________________________________________

_______________________________________________________

_______________________________________________________

Attachments

# Ball passing - Linear feed

## Supplementary material

- Table tennis balls

## Further information

[1]	Wikipedia: Slideway.