# Name: __________	Class: ___	Date: ________

# Model 3

# Room door

Note: The construction of the side of the door frame with the hinge pivot/hinges is the same as for the “garden gate” model and can be reused. The hinges on the door leaf are the same, too. The rest of the frame and the mechanism on the door are constructed in a different way than for the garden gate, however.

## Construction task

Build the room door model according to the building instructions. Note:

- The building blocks 7.5 of the door hinges must be offset by 2 mm to each other so that the axles can be inserted from above.

- The link 15 on the rear of the mechanism, which is connected to the door handle at the front, has to project a little way out of the building block 7.5.

Memorise these terms: The door handle is the element you have to press down to open the door. It moves the so-called falling latch (latch), which engages with the striker plate of the door frame when the door is closed.

Falling latch or

latch

Bolt

The bolt is a general term for the component that blocks the door when it is locked; it can be realised in many different ways.

It makes no difference whether the bolt is activated by a lock or pushed forward, closed, swivelled or hooked into place (garde gate).

The bolt can be a loose bar, a screwed-on hook or a slidable bolt, or a moving component of different types in a lock.

## Topic tasks

1. 	Hold the door open and press the door handle (the red link 15 on the front of the door). Try to describe what happens in individual steps during the mechanical process of pressing the door handle.

Follow the path of the force exerted on the door handle by the rotatable axle to the rear of the door. This is where the movable door “bolt” can be found.

a)	When the door is opened, the door handle is pressed ___________

b)	This makes the axle of the door handle turn ________. (Seen from the front)

c)	On the rear of the door, building block 7.5 with link 15 not quite inserted completely turns ________.

d)	The link 15 __________ the door bolt (the two angle girders 15 and the angular block 30° attached to these)

[ ] away from the door frame.

[ ] into the door frame

e)	This causes the spring (I-strut 45) to be ________.

f)	When the door handle is released, the ________.causes the door bolt to be pressed back into the __________.

2. 	Sometimes our self-built door might be able to be pulled open without the door handle being pressed. Which component has to be adjusted to stop this happening?

If the door opens so easily, the ____________________ must be moved further to the _______ in the direction of the door frame. The spring must press the angular block 30° reliably into the striker plate (the angle girder 30 in the door frame).

3. 	Close the door “loudly” by “slamming” i.e. without pressing the door handle down.

a)	Why does that work? Take a look at the door latch on the classroom door.

That works because the angular block 30° meets the door frame __________.

b)	Imagine our door cannot simply be pressed closed because the door bolt does not engage with the striker plate (angle girder 30 in the door frame) properly:

In this case, the ____________________ must be moved further to the ________.

## Experimental tasks

1.	Move building block 15 with the spring-loaded I-strut 45 on the rear of the door to cause the problems described in topic tasks 2 and 3. What conclusions do you draw?

_____________________________________________________

_____________________________________________________

_____________________________________________________

_____________________________________________________

2.	Find the position in which the door closes cleanly with the help of the door handle, but at the same time can simply be pushed shut smoothly.

3.	Remove the red flat “building block 15x30x5 with groove and pin” from the last construction step of the instructions on the rear of the door. Why does the door now no longer work properly?

_____________________________________________________

_____________________________________________________

_____________________________________________________

Attachments

# Room door

## Further information

[1]	Wikipedia: Door handle.