# Name: __________	Class: ___	Date: ________

# Model 5

# Sliding balance

Note: As well as fischertechnik parts, the weights of other small objects can be compared using the balance.

## Construction task

Build the sliding balance model according to the building instructions. Note:

- The clips must not be too tight against the components, so that all parts can move smoothly. This affects the upper balance beam support, the pan support and the support for the small weight underneath the slidable red building block 15 with bore.

- The small spacer ring 3 mm in the middle of the upper balance beam support (between the column and the balance beam) also contributes to smooth running - don’t forget to insert this.

## Topic tasks

1. 	Release the pan and move the so-called running weight or balance weight in such a way that the balance is in equilibrium. The fact that the S-rivet on the bottom of the pointer is aligned exactly with the S-rivet on the column of the balance shows that this is the case.

There is one point to which the balance weight must be moved to get the balance into equilibrium.

If you move the balance weight _______ to the fulcrum, the pan is lowered. If the balance weight is moved further away from the fulcrum, the pan _________.

2. 	Place a fischertechnik building block 15 in the pan. Now which direction do you have to move the balance weight in to get the balance into equilibrium??

[  ] the weight must be moved away from the fulcrum

[  ] the weight must be moved closer to the fulcrum

3. 	Repeat the same experiment using a building block 30. What do you find?

[  ] the weight has to be moved even further out

[  ] the weight must be moved closer to the fulcrum

The _________ the weight placed in the pan, the further the balance weight has to be moved ________.

## Experimental tasks

1.	Place a building block 30 in the pan and get the balance into equilibrium. Use a ruler to measure the distance of the balance weight from the balance fulcrum and note down this distance. If you don’t have a ruler, count how many holes there are in the horizontal beam of the balance between the fulcrum and the balance weight. Note the result for task 2.

Distance __________ cm or ________ holes

2.	Hold the balance weight exactly in the position noted in task 1. Now fill the pan with other weights (fischertechnik parts or other small parts) until the balance is in equilibrium again.

So if the position of the balance weight is not changed, we can measure whether weights of different objects...

[  ] are the same

[  ] are heavier

[  ] are lighter

In task 1, we used building block 30 to ___________ the weight to be measured by the sliding balance.