# Name: __________	Class: ___	Date: ________

# Model 8

# Ball passing 2 - Four-bar linkage

Note: You need at least one table tennis ball. You can use several at the same time, however, especially when several ball passing modules are built up connected to one another.

## Construction task

Build the model ball passing 2 according to the building instructions.

## Topic tasks

1.	Place a table tennis ball at the entrance (the shorter part of the track) and turn the crank clockwise (to the right). Observe how the gear wheel lifts the lever above the I-strut 45 and lowers it again, thus lifting the ball over the hill.

We have a combination of two types of gear in front of us:

a)	An “eccentric gear”

The axle with the I-strut 45 inserted in the gear wheel turns

[  ] outside the centre of the gear wheel

[  ] inside the centre of the gear wheel

This leads to a back-and-forth or up-and-down movement of the I-strut 45 away from the centre axle (axis of rotation) of the gear wheel. This type of movement is called eccentric.

b)	A “four-bar linkage”

Find the four rotating joints on the model, which are all connected to one another by lever:

- The centre of rotation of the gear wheel with the crank,

- the axle in the gear wheel with the rotating mounted strut (the “lever” here is the gear wheel itself),

- the rotating mount of the top of the strut in the long lever

- as well as the rotating support of the large lever at the model exit.

2.	If several modules are built up at the same time, connect them to form a longer track! All you have to do is insert the pins of building block 15 at the end of the module into the grooves of building block 15 at the entrance to the next module.

## Experimental tasks

1.	How many balls can one revolution of the crank lift?

_______________________________________________________

_______________________________________________________

2.	What is the angle girder 30 hanging downwards on the end of the lever for? What can happen if you leave this off?

_______________________________________________________

_______________________________________________________

_______________________________________________________

Attachments

# Ball passing - Four-bar linkage

## Supplementary material

- Table tennis balls

## Further information

[1]	Wikipedia: Eccentric.

[2]	Wikipedia: Four-bar linkage.

[3]	Wikipedia: Mechanical linkage.

[4]	Prof. Dr.-Ing. habil. Jürgen Dankert: Planar quadrilateral linkage.