# Name: __________	Class: ___	Date: ________

# Model 9

# Ball passing 3 – Launcher

Note: You need at least one table tennis ball. You can use several at the same time, however, especially when several ball passing modules are built up connected to one another.

## Construction task

Build the model ball passing 3 according to the building instructions. Observe the following points:

- All four building blocks 7.5 which the axle with the crank is attached to, have to be moved as far up as possible in the building blocks 30 so that the black gear wheel Z10 can engage cleanly with the red gear wheel Z20.

- The red link 15 must be fitted cleanly in the centre in clip adapter 20.

The “railing” is required for this model to stop the ball jumping out of the track to the left or right. It can also be attached to the three other ball passing models, but is not necessary on these.

## Topic tasks

1.	Place a table tennis ball at the entrance (the shorter part of the track) and turn the crank clockwise (to the right). You have to set (“adjust”) the mechanism at two points for it to work perfectly.

You can move the angular block 60°  (blue arrow) forwards or backwards in the model (to the right or left on the diagram) on building block 5 underneath.

The structural adapter girth  (green arrow) which the I-strut 45 is attached to, can be moved up or down. Start with the position shown on the diagram.

The rotating red link 15  must press the pin of the structural adapter girth at the end of the strut from the top downwards.

Adjust the mechanism in such a way that the table tennis ball bounces reliably over the hill but not higher than the railing.

Can you describe how the model works in your own words?

2.	If several modules are built up at the same time, connect them to form a longer track! All you have to do is insert the pins of building block 15 at the end of the module into the grooves of building block 15 at the entrance to the next module.

## Experimental tasks

1.	How often do you have to turn the crank for the link 15 to rotate once? Why is that, what happens here?

_______________________________________________________

_______________________________________________________

_______________________________________________________

2.	How many balls can be lifted when the crank moves through one revolution? Why do you not need two revolutions of the crank for one ball?

_______________________________________________________

_______________________________________________________

_______________________________________________________

Attachments

# Ball passing - Launcher

## Supplementary material

- Table tennis balls

## Further information

[1]	Wikipedia: Leaf spring (in: Spring).