# Name: __________	Class: ___	Date: ________

# Solution sheet model 7

# Truss building

## Topic task

- There are 12 structural struts in the building.

- There are 2 bay rails

## Experimental tasks

- c) The building is pressed together where the strut was.

- When the strut is fitted again, there is more resistance at this point, the shape of the building does not change.

- a) The diagonal strut also ensures that the shape of the building does not change. When it is removed, the building gives at this point.

- When the strut is fitted again, the building no longer gives at this point.

- Loads that can act on a building from the side are: Wind loads, moving loads or earthquake.

- Loads that can act on a building from above are: Dead weight, snow, wind and moving loads. With roofs, the so-called “man load” is also considered: there must not be anywhere on the roof that cannot bear the load of a person, otherwise the roof can collapse it if is stepped on.