# Name: __________	Class: ___	Date: ________

# Model 6

# Bridge

## Construction task

First construct the beam bridge according to the building instructions.

## Topic task

External statical determinacy

Find out whether the bridge is supported with external statical determinacy. Tip: consider the 2-dimensional schematic model of the bridge. Draw a sketch and enter the relevant forces on your sketch.

Sketch of static system beam bridge:

## Experimental task

- For the following experiment, replace the single-valued bearing (loose bearing / plain bearing) by the spring balance

and apply a load to the bridge in different places.

Enter the values measured at the spring balance depending on the distance sn in the following table. Do this experiment with 3 different weights. Note that the spring balance not only indicates the force resulting from the weight force of the load (FL), but also half of the dead weight of the bridge (130 gr / 2 = 65 gr)

| Distance sn | Bearing force Fn | Weight force FL |
| --- | --- | --- |
| s1 = 75 mm |  | 1 N |
| s2 = 150 mm |  | 1 N |
| s3 = 300 mm |  | 1 N |
| s1 = 75 mm |  | 2 N |
| s2 = 150 mm |  | 2 N |
| s3 = 300 mm |  | 2 N |
| s1 = 75 mm |  | 3 N |
| s2 = 150 mm |  | 3 N |
| s3 = 300 mm |  | 3 N |

Tip: To determine the weight force of the respective loads, you can weigh them using the spring balance and enter them directly in the table without having to determine the exact weight.

- Now build a through truss and a deck truss one after the other on the beam bridge according to the building instructions.

Determine whether the upper longitudinal girder of the through truss is subject to tension or compression. And what about the deck truss?

________________________________________________________

________________________________________________________

________________________________________________________

________________________________________________________

3. 	What is the deciding factor as to whether a bridge is built with a through truss or a deck truss?

________________________________________________________

________________________________________________________

________________________________________________________

________________________________________________________