# Barrier with double-acting cylinder - throttling a double-acting cylinder in only one direction

Stefan Falk

The “Barrier” model built for the previous task is also used for this task.

## Topic

We will be showing how you can throttle the movement of a double-acting cylinder in just one direction, or throttle it at different strengths in both directions.

## Learning objective

- By connecting a throttle and check valve in parallel, the throttle can be relevant in only one direction; it can simply be bypassed in the other direction by the check valve.

- This allows the compressed air to be throttled in just one direction of flow, while it practically flows freely in the other.

- In this way, we can throttle the exhaust on one side of the cylinder, while allowing intake air into the cylinder without throttling. This creates directionally-dependent throttling.

## Time required

45 min, if the barrier from the previous task is still built.