Solution sheet 11

# Transistor curve

The solution sheet shows an example of one possible solution for the task. The only specification is the wiring diagram. Therefore, many different solutions are possible. Ensure that the design still reflects the specifications of the wiring diagram.

## Example solution for construction task

The construction task can be solved using the building instructions.

## Example solution for topic task

1. The results of the tasks under points 1 to 5 will produce a table of measured values. An example of these results is shown below.

...

6. Transfer the measured values into the diagram below. You will get the transmission curve for the transistor.

An example result is provided below.

7. What conclusions can we draw about the correlation between base and collector current as well as the amplification factor?

Current only flows after about potentiometer setting 4. The diffusion voltage between the base and emitter is exceeded here.

Between positions 4 and 6, the collector current increases with the base current in a relatively even manner. The current amplification factor is at a level set by the manufacturer.

Above a setting of roughly 6 ½, the collector current will not increase further, even if the base current is increased. The transistor is operated at saturation.

Example result of measurements

| Potentiometer | Base current IB | Collector current IC | Amplification B |
| --- | --- | --- | --- |
| 0 | 0.0µA | 0.0mA | - |
| ½ | 0.0µA | 0.0mA | - |
| 1 | 0.0µA | 0.0mA | - |
| 1 ½ | 0.0µA | 0.0mA | - |
| 2 | 0.0µA | 0.0mA | - |
| 2 ½ | 0.0µA | 0.0mA | - |
| 3 | 0.0µA | 0.0mA | - |
| 3 ½ | 0.0µA | 0.0mA | - |
| 4 | 2.5µA | 0.6mA | 240 |
| 4 ½ | 27.6µA | 7.4mA | 268 |
| 5 | 50.4µA | 13.4mA | 266 |
| 5 ½ | 72.5µA | 18.5mA | 255 |
| 6 | 88.0µA | 21.5mA | 244 |
| 6 ½ | 107.9µA | 22.4mA | 208 |
| 7 | 118.0µA | 22.6mA | 192 |
| 7 ½ | 139.9µA | 22.9mA | 164 |
| 8 | 154.5µA | 23.0mA | 149 |
| 8 ½ | 168.8µA | 23.1mA | 137 |
| 9 | 185.9µA | 23.1mA | 124 |
| 9 ½ | 202.5µA | 23.1mA | 114 |
| 10 | 216.9µA | 23.1mA | 107 |

Current amplification curve