Solution sheet 3

# OR circuit

DIN 40700

The solution sheet shows an example of one possible solution for the task. The only specification is the wiring diagram. Therefore, many different solutions are possible. Ensure that the design still reflects the specifications of the wiring diagram.

Theoretical background:

Figure 1: Wiring symbols DIN (outdated) and IEC

Today, circuits are used in computer technology that consist of billions of components on a single tiny piece of silicone. The active components are transistors that are connected to create logic gates. The function of these gates can be demonstrated easily using switches or the buttons included in this experimental kit.

An OR gate is one type. It consists of at least two inputs and one output. The inputs in this experiment are the two buttons. When they are activated, the LED lights up according to the following diagram.

| Button 1 | Button 2 | LED |  | A | B | Y |
| --- | --- | --- | --- | --- | --- | --- |
| off | off | off |  | 0 | 0 | 0 |
| on | off | on |  | 1 | 0 | 1 |
| off | on | on |  | 0 | 1 | 1 |
| on | on | on |  | 1 | 1 | 1 |

The second table shows the same result, described in a more general way. It is called a truth table. There are two inputs, A and B, as well as one output Y. The states of the inputs and output are indicated using a binary code of ones and zeroes.

## Example solution for construction task

The construction task can be solved using the building instructions.

## Example solution for topic task

1. What happens when any one button is pushed?

The LED is illuminated.

2. Why is the LED illuminated?

No matter which button is pressed, the circuit is closed and the LED is illuminated.

3. What happens when both buttons are pushed?

The LED is illuminated.

4. Why is the LED illuminated?

The circuit is also closed if both buttons are pressed. The LED is illuminated.

## Evaluating the experimental task

| Button 1 | Button 2 | LED |
| --- | --- | --- |
| off | off | off |
| on | off | on |
| off | on | on |
| on | off | on |

- Two switches are connected in parallel in the circuit. The circuit can be closed if one of the two switches is closed. This is very easy to see in the wiring diagram. In the following table, write in the state of the LED depending on the switching position of the button.

The results are as follows. The LED lights up no matter which button is pressed. The LED also lights up if both buttons are pressed.

Figure 2: Exclusive OR circuit

Can you modify the circuit so that the LED does not light up if neither or both of the buttons are pressed? Complete the table.

The circuit must be built as shown in Figure 2. The centre contact of the first button is connected to the battery holder. The centre contact of the second button is connected to the LED. The two switching contacts of the button are connected to one another in a crosswise fashion. The closed contact of the first button is connected to the open contact of the other button. This creates an exclusive OR circuit (XOR). The circuit symbol for the logic gate is shown in Figure 3.

The truth table looks like the following:

| Button 1 | Button 2 | LED |
| --- | --- | --- |
| off | off | off |
| on | off | on |
| off | on | on |
| on | off | off |

Figure 3: Exclusive OR wiring symbol