Task 3

# OR circuit

## Construction task

Build the electronic circuit as shown in the wiring diagram in Figure 1. Make sure to check for correct polarity of the LED. The LED has a white plus sign on the black light module.

Figure 1: Circuit

Try to build the circuit without the building instructions.

## Building tips

Make sure to check for correct polarity of the LED. The LED has a white plus sign on the black light module.

Figure 2: Battery holder

Connect the battery holder as shown in Figure 2. The voltage supply is connected with correct polarity if the switch is pressed in the direction of the arrow.

## Topic task

Turn on the supply voltage using the switch on the battery holder. Then press the button.

What happens when any one button is pushed?

Why is the LED illuminated?

What happens when both buttons are pushed?

Why is the LED illuminated now?

## Experimental task

| Button 1 | Button 2 | LED |
| --- | --- | --- |
| off | off |  |
| on | off |  |
| off | on |  |
| on | off |  |

- Two switches are connected in parallel in the circuit. The circuit can be closed if one of the two switches is closed. This is very easy to see in the wiring diagram. In the following table, enter the state of the LED depending on the switching position of the button.

| Button 1 | Button 2 | LED |
| --- | --- | --- |
| off | off |  |
| on | off |  |
| off | on |  |
| on | off |  |

- Can you modify the circuit so that the LED does not light up if neither or both of the buttons are pressed? Complete the table.