# Task 2: Brake assistant, cruise control and lane departure warning system

The vehicle is now equipped with its first autonomous capabilities (driver assistance systems). If there is an obstacle, it should initiate a braking procedure (braking assistant), maintain a specified speed (cruise control), and independently follow a track (lane departure warning system).

## Topic

Driver assistance systems (braking assistant, lane departure warning system and cruise control), sensors (evaluating encoder pulses, ultrasound values and camera images for speed control, detecting obstacles and travel lanes) and controllers (P and PD controllers).

## Learning objectives

Distance measurement using the ultrasound sensor

Analogue proportional control with speed measurement (cruise control) and using a camera with image evaluation (lane departure warning system)

PD controller (lane departure warning system)

## Time required

The servo motor and USB camera are connected to the autonomous vehicle.

The task builds on the “obstacle detector” (ultrasound distance measurement) and “lane tracker” (proportional controller) from the tasks in the Robotics TXT 4.0 Base Set. Depending on their age and previous level of knowledge, students will need 180-270 minutes (four to six hours of instruction) to develop the programs needed to solve the programming tasks. The functional principle of a proportional controller should be addressed in instruction in advance.

Highly skilled or older students could be given the experimental tasks (lane departure warning system with P and PD controller). These will take an additional approx. 90-180 minutes to solve (two to four hours of instruction).

Annex

# Task 2: Brake assistant, cruise control and lane departure warning system

## Required materials

PC for program development, local or via web interface.

USB cable or BLE or WiFi connection to transmit the program to the TXT4.0.

Travel route with markings on the enclosed sheet (or as a print out of the travel route file)

Obstacle (such as a book or a box)

## Further information

[1]	Jim Meininghaus: Die Geschichte des Tempomaten. Wie ein Blinder das Autofahren veränderte. 03.03.2014, motor-talk.de.

[2]	Thomas Paulsen: Autonomes Fahren: Die 5 Stufen zum selbstfahrenden Auto. 07.11.2018, adac.de.

[3]	Online diagram editor for creating state diagrams (Format drawio): https://www.diagrammeditor.de/