# Name: __________	Class: ___	Date: ________

# Task 3

# Automatic light

In road traffic, the driver assistance systems in our vehicle could cause an accident – the vehicles following it would not have any turn indicators or flashing lights to warn them. Because of this, the vehicle will now be equipped with lighting functions.

## Construction task

Connect the two front headlights (low beams) parallel to output O5 and the reversing light (white) to output O4. The brake light (red) is connected to O3. The left turn indicator is connected to O7, and the right to O8.

## Programming tasks

Now, you will add automatic lighting functions to your control program with the driver assistance systems.

1. Reversing light

The reversing light should light up when the motor turns backwards. Add a thread to your control program for this purpose.

2. Brake light

The brake light should light up when the motor slows down.

2a. Add a thread to your program for this purpose.

2b. The brake light should remain illuminated for 0.5 seconds to ensure that it can be seen by the following vehicle, even with a short delay.

2c. Include the strength of the delay in your brake light function. The greater the braking power, the brighter the brake light should be illuminated.

3. Turn indicators

The two turn indicators should be activated automatically when the steering angle exceeds a certain value (here: servo lever at a position smaller than 190 or greater than 310). Please note: According to the Road Traffic Regulations, a turn indicator must flash at a frequency of 1-2 Hz.

The information on whether the turn indicator is activated should be exchanged between the main program and thread via a semaphore (here: a commonly used status variable).

## Experimental tasks

1. Warning flasher

A warning flasher should be activated in case of an “emergency stop” or braking due to an obstacle.

Add this function to your turn indicator thread.

2. Low beam

The front headlight and tail light should be automatically activated when it becomes dark (see also task 6 in the Robotics TXT 4.0 Base Set).

To do so, attach the phototransistor to your vehicle and connect it to input I7.

Program the automatic light function as an additional thread.

Since the tail light is also used as the brake light, you may only activate it at half brightness. Adjust the automatic brake light so that the tail light is illuminated brighter during braking (see programming task 2c top).

Annex

# Task 3: Automatic light

## Required materials

PC for program development, local or via web interface.

USB cable or BLE or WiFi connection to transmit the program to the TXT4.0.

## Further information

[1]	Wikipedia: Nebenläufigkeit.