# Name: __________	Class: ___	Date: ________

# Task 2

# Track follower

## Construction task

For the third programming task and the experimental tasks, we will need the camera on the Mecanum vehicle base model, with sensors. Connect it to the USB1 port of the TXT.

Important note: When the TXT is started, the servo automatically switches to “forward setting”. Then, insert the servo lever so that the camera is tilted at an angle of approximately 45° from vertical to the front.

Note: If you move the servo manually with the TXT switched on, you may damage it.

## Programming tasks

1. Track follower with track sensor

Now, the Mecanum vehicle should follow the black, approx. 2 cm wide track on the course from the Robotics TXT 4.0 Base Set (see also task 8 from the Robotics TXT 4.0 Base Set) with the help of the track sensor.

1a. First, create a state diagram that describes the possible states and behaviour of the Mecanum vehicle.

1b. Convert your state diagram into a Blockly program. To do so, use a state variable with a state value which you determine based on the values from the IR sensors.

Test the program on the circular course of the Robotics TXT 4.0 Base Set.

1c. How can you increase the speed of the Mecanum track follower? Conduct tests and measure the time the track follower needs to complete the course.

| Solution variants (adjustments, parameters) | Time |
| --- | --- |

2. Track follower with obstacle detection

There can also be obstacles on the track. If the ultrasound sensor detects an obstacle at a maximum 2 cm distance on front of the vehicle, the vehicle should move sideways to travel past the obstacle and then find the track once again.

Think of different possible solutions to find the line. Add this function to your Blockly program and test the variants. What advantages and disadvantages do they have?

3. Track follower with colour control

You will see four coloured surfaces at the corners of the course. You can use these coloured surfaces to give commands to your Mecanum vehicle.

3a. Add a colour recognition function to your Blockly program. Output the RGB-HEX colour code of the detected colour on the console in order to calibrate the detection.

3b. Determine how the Mecanum vehicle should react to different colours (at least two), and modify your Blockly program accordingly.

Note: Do not be surprised if the HEX value displayed on the console seems to fluctuate significantly – the hue value is stable. You can use any of the displayed HEX colour codes for the detection.

## Experimental tasks

1. Track follower with proportional controller

Now, you should equip the track follower with a proportional controller (P controller) like the track follower buggy from task 8 of the Robotics TXT 4.0 Base Set. The amount by which the direction is corrected should depend on the amount of line deviation. Here as well, the camera's line detection function should be used to determine deviation from the track.

1a. First, label the following control circuit with the values for your Mecanum track follower:

Controlled system

Controller

1b. Design and program a proportional controller (P controller) for your Mecanum track follower. You can use the detection process for the black line from experimental task 1 from task 8 of the Robotics TXT 4.0 Base Set.

Test your program on the simple straight line course by starting the Mecanum track follower in parallel to the track, so that the centre point of the track appears far to the left of the image.

Tip: Start with the proportionality factor kp=2. Increase the value in increments of 1 until the Mecanum track follower quickly “engages”, meaning that the deviation from the track quickly reduces without the controller jumping or overshooting too severely.

1c. Add a text output to your program that indicates

- the time (in ms) which has passed since the start of the program and

- the value for the current deviation

separated by a space on the console after each change in the deviation from the track.

After each test run with a different value for kp, copy the information output on the console into a spreadsheet, and display the values graphically in a diagram (x: Time, y: Deviation from the track). Adjust the proportionality factor kp until the curve progression engages quickly.

Test the track follower with the circular course from the Robotics TXT 4.0 Base Set. You may need to reduce the maximum speed somewhat.

2. Track follower with PD controller

Just like the buggy in task 8 of the Robotics TXT 4.0 Base Set, you can add a “D” factor (differential factor) to the controller that takes the amount of change in deviation from the track into account in the speed correction, to further dampen overshooting.

Expand the P controller in your Mecanum track follower from experimental task 1 so that it is a PD controller. Complete test runs with different values for the differential factor kd and use a spreadsheet program to display the data graphically.

Tip: Start your tests with the differential factor kd=0.25 and increase its value in increments of 0.25 until the overshoot of the P controller is dampened.

Annex

# Task 2: Track follower

## Required materials

PC for program development, local or via web interface.

USB cable or BLE or WiFi connection to transmit the program to the TXT4.0.

Course sheet with straight black line 2 cm wide

Course sheet with black line in a closed circle, 2 cm wide (from the Robotics TXT 4.0 Base Set)

Obstacle (box, can, ...)

## Further information

[1]	FRC Team 2605 (Bellingham, WA): How a Mecanum Drive Works. github.io

[2]	Wikipedia: Finite automaton (state machine)

[3]	Ferdinand Wagner, Ruedi Schmuki, Thomas Wagner, Peter Wolstenholme: Modeling Software with Finite State Machines. A Practical Approach. Auerbach Publications, 2006.

[4]	Online diagram editor for creating state diagrams (Format drawio): https://www.diagrammeditor.de/

[5]	Wikipedia: Control technology.

[6]	Wikipedia: Controller.

[7]	RN-Wissen: Control technology.

[8]	Tim Wescott: PID without a PhD. Embedded Systems Programming, 10/2000, p. 86-108.