# Name: __________	Class: ___	Date: ________

# Solution sheet task 4

# Parking assistant

## Programming tasks

1. Find a parking space

The parking procedure can be completed easily by hand with the servo lever removed. The easiest way to determine the length of the required parking space is to mark it in pencil on the track and measure it (see figure).

Parallel_Parking_Model.jpg

Width of the parking space: The parking space must be at least as wide as the vehicle, or b = 14.5 cm, plus 2 cm to the boundary of the track.

The space should also be an additional 1.5 cm wider, since the tires extend past the width of the vehicle at a steering angle.

If the ultrasound sensor is mounted so that it measures from the exterior edge of the vehicle, then another 2 cm must be added to the side line, as well as its width (2 cm).

Therefore, there is a sufficiently wide space to the right of the vehicle when the sensor measures at least 22 cm to the closest object.

Length of the parking space: When parking in two manoeuvres, the centre point of the differential must first move backwards and to the right around 17.5 cm at the maximum steering angle, before it crosses the centre of the side line, then move another 17.5 cm at the maximum steering angle to the left. The length of the rear of the vehicle (5.5 cm, measured from the centre of the differential to the exterior end of the vehicle) must be added to this distance of 35 cm, and the “starting distance” of 2 cm (between the position of the differential when the parking procedure is started and the point at which the left front wheel crosses the side strip) must be subtracted. Therefore, the required minimum length of the parking space adds up to

I=38.5 cm

The minimum length of the parking space I can be used to calculate the number of impulses required for this distance i based on the following formula (see task 1):

i=I·120.5·137·63.9 1cm≈I· 5.789 1cm

If we consider the adjusted conversion factor from the test in task 1, the conversion is corrected to

i=I·1000.0017≈I· 5.882 1cm

Therefore, the parking space length we are looking for I=38.5 cm requires i≈227 impulses.

Scatter of the ultrasound signal: If there are obstacles at a distance of 4 cm to the right beside the track boundary, the distance to the ultrasound sensor is 8 cm. Due to the scattering angle of the ultrasound sensor, the parking space is not detected until the centre of the ultrasound sensor already extends 3 cm into the parking space. The end of the parking space is accordingly detected 3 cm in front of the position where the centre of the sensor reaches the exterior edge of the obstacle. Therefore, if the sensor measures a width of at least 22 cm over a distance of 32.5 cm (or i'≈191 impulses), then the parking space it has found is long enough.

For the measurement of the length of the parking space, it is irrelevant where the ultrasound sensor is attached on the side of the vehicle. Therefore, the vehicle must stop in order to start the parking procedure so that the rear axle (or the differential) is exactly 2 cm behind the end of the parking space (the exterior edge of the obstacle). To do so, after the ultrasound sensor has detected the end of the parking space,

- it must travel an additional 3 cm (by which the end of the parking space was detected too early)

- plus the above 2 cm

- in addition to the distance from the centre of the ultrasound sensor to the rear axle

. For example, if the ultrasound sensor is mounted 7 cm in front of the rear axle, the vehicle must travel an additional 12 cm (or 71 impulses) after detecting the end of the parking space.

1b. State diagram:

State-Transistion_Diagram_Find_Parking_Space.drawio

1c. Program (example) finding a parking space:

Find_Parking_Space.ft

2. Parking manoeuvres

2a. The easiest way to determine the impulses travelled is using the interface test: The vehicle is parked manually; the impulses delivered by the encoder are read and noted in the interface test. With the steering turned all the way to the right and left, 125 impulses in each direction are required to complete the procedure.

2b. For the vehicle to be in the centre of the required parking area of the parking space at the end, it must travel forward 5.75 cm after completing the procedure, corresponding to around 34 impulses.

2c. Program (example) parking manoeuvre:

Parking_Manoevre.ft

The variables “backward_distance” and “forward_distance” contain the impulses to be travelled (125 or 34). “minangle” and “maxangle” are the maximum possible turning angles of the servo in the servo holder (see task 2); they are around 130 and 375. 250 ms is a good choice for the short pause after a steering angle (“pause”).

## Experimental tasks

1. Parking assistant

One solution to the task is summarized in the Autonomous_Parking.ft program.

The turn signal may not be activated automatically; otherwise it will switch to the left signal in the middle of the parking procedure.

2. Calculating the parking procedure

The centre point of the differential moves exactly along two segments of a circle during the parking process of the length of s. The centre point M of the first circle corresponds to the centre point of the turning radius in our Ackerman steering: The extensions of the front two axle stubs of the steering and rear wheel axle meet at precisely this point.

Mathematical_Model_Parallel_Parking.jpg

We will now calculate the length of this segment of the circle s. This will tell us how many encoder impulses we will need to travel this segment. The following applies: s is to the circumference of the circle (=2π∙r) as δ is to 360°, therefore:

s=π∙δ∙r180°

We can express the angle δ by the radius r, for in the right triangle that is formed by M and the two intersecting points of the circle segment lines with the side line, the following applies:

cosδ=(r-d)r

For the distance d from the centre point of the axis to the (centre of the) side line, we will assume that the vehicle is located around 2 cm to the left of the track boundary. Then, the following applies for a vehicle width of b = 14.5 cm and our 2 cm wide side line:

d=10.25 cm

We calculate the radius r of the turning circle based on the triangle formed by M and the two centre points of the front and rear axles of the vehicle. We designate the centre distance with a:

tanα=ar , therefore: r=atanα

The angle α corresponds to the (maximum) steering angle of our servo lever. With our vehicle, the steering angle is around 38° in each case. We can measure the centre distance: a=15.5 cm.

Finally, the result is (with α=38°):

s=π180°∙arccosa-d∙tanαa∙atanα≈21.15 cm

This corresponds to around 122 impulses.

Now, we can also calculate the required length of the parking space. According to the Pythagorean theorem:

r2=t22+r-d2

Therefore (with α=38°):

t=2∙atanα2-atanα-d2≈34.73 cm

We must also add the length of the rear of the vehicle to this value (5.5 cm) and subtract 2 cm which we will travel beyond the parking space. Therefore, our parking space should be at least 38.23 cm long - this corresponds fairly exactly to the value we determined through experimenting.

Annex

# Parking assistant

## Required materials

PC for program development, local or via web interface.

USB cable or BLE or WiFi connection to transmit the program to the TXT4.0.

Test travel route

## Further information

[1]	VDA: Automatisierung: Von Fahrerassistenzsystemen zum automatisierten Fahren. September 2015.

[2]	Nina Tetzlaff: Autonome Straßenfahrzeuge. Erfinderaktivitäten 2018/2019, DPMA, p. 62-81.

[3]	VW: Park Assist Steering 2.0. Design and function. Service Training, 2021.