# Name: __________	Class: ___	Date: ________

# Task 4

# Parking assistant

Now, we will equip the vehicle with another important driver assistance system: a parking assistant. This will allow the vehicle to independently find a parking space of sufficient size, then park in it backwards in two manoeuvres.

## Construction task

Mount the ultrasound sensor on your vehicle so that it can measure the distance between the vehicle and objects on the right side of the street.

## Programming tasks

Configuring the sensors:

1. Find a parking space

Now, you should develop a program that steers the vehicle at a distance of approx. 2 cm along the right edge of the road and uses an ultrasound sensor to look for a sufficiently large parking space. You can use objects (such as boxes) to define the parking spaces. At the end of the first sufficiently large space, the vehicle should stop and begin with the parking procedure.

First, manually determine how large (how long and how wide) a parking space must be for your vehicle to be able to park in it backwards. Take the scatter of the ultrasound signal into consideration: When does the sensor detect the start and end of the parking space if it is bound by objects located around 2 cm to the right beside the edge of the road.

Then, determine how far the vehicle must stop behind the end of the parking space so it can start the parking process.

1a. Illustrate your parking procedure and measurements using a drawing.

1b. Sketch a state diagram.

1c. Program the vehicle to search for a parking space. Use the lane departure warning system from task 2 to do so.

Test your program on the road section on the enclosed sheet with obstacles set up at different distances to the right beside the edge of the road.

2. Parking manoeuvres

In the second step, the vehicle should park in the space it has found by moving backwards in two manoeuvres.

2a. First, determine manually (interface test) how many impulses the vehicle must complete first with the steering angle to the right and then with the steering angle to the left.

2b. How many impulses are still required for the vehicle to come to a standstill in the centre (of the part required for parking) of the parking space?

2c. Program the parking procedure and test it on the road section.

## Experimental tasks

1. Parking assistant

Now, add the functions developed in task 3 to your parking assistant, so that it fulfils all requirements of the Road Traffic Regulation during parking (brake light, reversing light, turn indicators, low beams). Add a “success signal” to your program that will sound after the parking procedure is complete, and make the warning flashers light up five times as well.

2. Calculating the parking procedure

You can also calculate the parking procedure: The centre of the differential describes two segments of a circle, the radii of which you can derive from the steering angle.

2a. Draw the phase of the parking procedure when the vehicle is driving backwards.

2b. Use the angles and several sets to calculate the number of impulses required based on the correct distances to the side line.

Compare the result with the values you determined by experimenting in the programming task.

Annex

# Parking assistant

## Required materials

PC for program development, local or via web interface.

USB cable or BLE or WiFi connection to transmit the program to the TXT4.0.

Test travel route

## Further information

[1]	VDA: Automatisierung: Von Fahrerassistenzsystemen zum automatisierten Fahren. September 2015.

[2]	Nina Tetzlaff: Autonome Straßenfahrzeuge. Erfinderaktivitäten 2018/2019, DPMA, p. 62-81.

[3]	VW: Park Assist Steering 2.0. Design and function. Service Training, 2021.