# Name: __________	Class: ___	Date: ________

# Solution sheet task 2

# Track follower

The students’ solutions should use the sub-functions from task 1 to control the vehicle. Just as in the solution for task 8 in the Robotics TXT 4.0 Base Set – a state variable should be used to differentiate between the states when programming the track follower. This makes the programs very clear and easy to understand.

The two experimental tasks help students understand how to develop and configure a P and PD controller. The output and graphic display of measured values are particularly important.

## Construction task

Connecting the sensors:

## Programming tasks

1. Track follower with track sensor

Just like the buggy in task 8 of the Robotics TXT 4.0 Base Set, the Mecanum vehicle should be steered so that both IR sensors in the track sensor are centred over the black line, so that they both deliver a value of 0.

1a. State diagram for the (digital) track follower:

State-Transition_Diagram_Line_Follower.drawio

1b. Depending on the values from the left and right IR sensors, the state variables state have the following values:

- state = 0 → on the track (both sensors deliver a value of 0)

- state = 1 → track to the left (only the left sensor has the value 0)

- state = 2 → track to the right (only the right sensor has the value 0)

- state = 3 → track lost (both sensors deliver the value 1)

|  | I8 (left sensor ) | I7 (right sensor) |
| --- | --- | --- |
| Black line (track) | 0 → state += 0 | 0 → state += 0 |
| Light area | 1 → state += 2 | 1 → state += 1 |

Reading example: If the left sensor (I8) delivers the value 0 and the right sensor (I7) delivers the value 1, then the track is located to the left of the centre of the vehicle. The state variable state has a value of 1, and in this state the vehicle must be steered to the left so that the sensor is centred over the track once again.

The following example solution uses the navigation functions programmed in task 1 for the Mecanum vehicle. This makes the program very easy to understand.

At the start of the loop, the state variable is set to a state value from {0, 1, 2, 3} depending on the values of the two IR sensors.

Program excerpt (example):

Mecanum_Line_Follower_digital.ft

1c. The speed of the track follower can be increased by choosing the highest possible main speed (“speed”) and reducing the speed distance between the motors during steering, adjusting the “slow” speed so that the track follower does not lose the track, particularly on curves.

Note: You can also understand the digital track follower as a three-point controller which regulates the speed of the motors depending on the three states “left of the track”, “on the track” and “right of the track”. The area within which both IR sensors deliver the target value of “0” (both are directly over the track) is also called the hysteresis.

2. Track follower with obstacle detection

After the vehicle deviates from the line, it can find it again behind the obstacle by

- driving to the side once again (disadvantage: the depth of the obstacle is unknown, therefore it may hit the obstacle or miss the line if there is a tight curve right behind the obstacle; advantage: the vehicle finds a straight line in the right position)

- travelling diagonally (disadvantage: it may hit the obstacle; it is very likely to miss the line; advantage: a straight line is found in the correct alignment to continue driving)

- turning by 45°-90°, then driving straight ahead (disadvantage: it may hit the obstacle, miss the line or continue driving in the wrong direction once it finds the line)

- Driving in a curve around the obstacle (disadvantage: the vehicle may continue driving in the wrong direction once it finds the line)

Program excerpt (example):

…

Mecanum_Line_Follower_Obstacle_digital.ft

3. Track follower with colour control

Connecting the camera:

3b. Program excerpt (example):

Mecanum_Line_Follower_with_Color_Detection_digital.ft

## Experimental tasks

Connecting the sensors:

1. Track follower with proportional controller

The camera line detector delivers the deviation from the centre of the detection window.

Configuring the line detection:

Since coloured surfaces can be detected as a line, the line detection should be set to at least two lines.

1a. Control circuit:

y: Position of the track centre point in the camera image

Controlled system

u: Speed of the steering motors

Controller

e: Deviation of the track from the centre of the image

w: Specification: Track in the centre of the image

d: Curve

Regelkreis_Mecanum_Spurfolger.drawio

1b. Program excerpt (example):

Mecanum_Line_Follower_P_Controller.ft

Mecanum_Line_Follower_P_Controller.ft

Multiplied by the proportionality factor kp, the position is added to or subtracted from the motor speed for turning left or right (function turn), so that the deviation from the track is reduced with the change in the direction of travel. Depending on the vertical position of the line detector in the camera configuration, it may be necessary to adjust the minimum and maximum line width in the program.

1c. Measurement results for the P controller (with kp∊{2, 3, 4, 5, 6, 7, 8}):

Mecanum_Line_Follower_with_P_Controller_Results.jpg

The red line with kp=6 engages most quickly after approx. 1.7 seconds, without overshooting a second time. The controller oscillates with kp=8 (dark blue line).

2. Track follower with PD controller

The D factor of the PD controller is the change in deviation multiplied by the differential factor kd.

Program (example):

Mecanum_Line_Follower_with_PD_Controller.ft

Measurement results for the PD controller (with kd∊{0, 0.25, 0.5, 0.75, 1, 1.25, 1.5, 1.75, 2, 2.25, 2.5}):

Mecanum_Line_Follower_with_PD_Controller_Results.jpg

The differential factor kd=1.75 was most effective in dampening overshoot in the completed tests. The value can differ depending on the position of the line detection in the camera image and small structural differences between models.

Annex

# Task 2: Track follower

## Required materials

PC for program development, local or via web interface.

USB cable or BLE or WiFi connection to transmit the program to the TXT4.0.

Course sheet with straight black line 2 cm wide

Course sheet with black line in a closed circle, 2 cm wide (from the Robotics TXT 4.0 Base Set)

Obstacle (box, can, ...)

## Further information

[1]	FRC Team 2605 (Bellingham, WA): How a Mecanum Drive Works. github.io

[2]	Wikipedia: Finite automaton (state machine)

[3]	Ferdinand Wagner, Ruedi Schmuki, Thomas Wagner, Peter Wolstenholme: Modeling Software with Finite State Machines. A Practical Approach. Auerbach Publications, 2006.

[4]	Online diagram editor for creating state diagrams (Format drawio): https://www.diagrammeditor.de/

[5]	Wikipedia: Control technology.

[6]	Wikipedia: Controller.

[7]	RN-Wissen: Control technology.

[8]	Tim Wescott: PID without a PhD. Embedded Systems Programming, 10/2000, p. 86-108.