# Name: __________	Class: ___	Date: ________

# Task 1

# Mecanum vehicle with obstacle detection

## Construction task

Build the Mecanum vehicle base model with sensors, according to the building instructions. The hubs of the Z20 toothed gears and the Mecanum wheels must be tightened well so there is no slippage while the vehicle is driving.

In this task, you will use the track sensor (front) and ultrasound distance sensor for obstacle detection.

The ultrasound distance sensor is connected to I6 (black cable), the two infrared (IR) sensors are connected to I7 (right sensor), and the yellow/blue cable to I8 (left sensor, blue cable). The IR sensors and the ultrasound sensor must also be supplied with power via the 9V voltage output of the TXT.

Use the interface test to check whether the four motors are connected correctly (forward: anti-clockwise), that the four encoders are connected to the correct counter inputs (M1: C1, M2: C2, M3: C3 and M4: C4) and the two IR sensors are delivering the right values (white surface: 1, black line: 0).

## Programming tasks

Most important types of movement with Mecanum wheels

1. Synchronous drive in all directions

The figure shows the control of a Mecanum vehicle. The top three types of movements show the wheel drive required for travelling straight ahead, travelling at an angle and travelling sideways. Each of these three types of movement includes the forward and backward movement, as well as the movement to the left and right. If the black arrow beside the wheel is pointing upwards, the associated wheel should turn forwards.

Because of this, we can differentiate between the following eight movements of the Mecanum vehicle (see also the animation in [1]):

- Forwards

- Backwards

- Sideways to the left

- Sideways to the right

- At an angle to the front left

- At an angle to the front right

- At an angle to the rear left

- At an angle to the rear right

Develop a Blockly subroutine for each of these movements (a function) to which you can transmit the speed of the motors as a parameter.

The functions will be required in subsequent functions. These make the control programs clearer and easier to understand. Test your functions using simple example programs.

Please note: Synchronisation of the motors is particularly important in Mecanum wheels (see also task 6 of the Robotics TXT 4.0 Base Set).

2. Synchronous turning

The bottom two types of movement shown in the figure are the two most important rotational movements that can be completed with Mecanum wheels: turning in place and travelling along a curve. These also include multiple directions of movement, such as rotating to the left and right, or turning left or right and travelling forwards and backwards along a curve.

In total, this results in six additional movements:

- Rotating in place to the right (clockwise)

- Rotating in place to the left (anti-clockwise)

- Turning to the right

- Turning to the left

- Turning backwards to the right

- Turning backwards to the left

Develop a Blockly subroutine for each of these movements (a function) to which you can transmit the speed of the motors as a parameter. Test your functions using simple example programs.

3. Synchronous drive with distance specification

To navigate the Mecanum vehicle precisely, you will now need a section of the command set from programming tasks 1 and 2, each with a specified distance as well – the number of impulses by which the motor should turn.

3a. Add a specified distance to each of the following movement functions from programming tasks 1 and 2: a set number of impulses

- To travel straight ahead (forward/backward)

- To travel sideways (left/right) and

- To turn around the vehicle's own axis (left: clockwise, right: anti-clockwise).

3b. Test these six functions on a previously measured test track and use experiments to determine the factors (impulses per cm or impulses per °) to calculate the required impulses from

- A distance travelling straight ahead, in cm

- A distance travelling sideways, in cm and

- A rotational angle indicated in °

Similar to task 6 in the Robotics TXT 4.0 Base Set, you can measure the circumference of a Mecanum wheel as an initial estimate of the conversion factor when travelling straight ahead, and use it to derive the factor.

4. Line detection

The Mecanum vehicle should now identify the boundary lines and edges using the track sensor, and avoid these: If a (black) boundary line or gap is detected, the vehicle should drive back 10 cm, turn a quarter turn away from the edge (45°) and continue travelling.

4a. Draw a state diagram showing this.

4b. Now, use your navigation functions from programming tasks 1 and 2 to write a Blockly program (see also task 6 from the Robotics TXT 4.0 Base Set).

## Experimental tasks

1. Obstacle detection using ultrasound

The Mecanum vehicle is equipped with an ultrasound sensor which delivers the distance from an object in cm (see also task 1 from the Robotics TXT 4.0 Base Set).

Write a Blockly program that prevents the vehicle from coming any closer than 15 cm to an obstacle. When it detects an obstacle, it should avoid it by driving sideways until the ultrasound sensor no longer detects an obstacle within a distance of 25 cm (see also task 6 in the Robotics TXT 4.0 Base Set), and then continue travelling.

2. Encoder navigation

You can also use the values from the encoder in the Mecanum vehicle to calculate the distance travelled. This will allow you to navigate to a specified goal, for which the linear distance in cm is specified. Mark a goal 3 m away (linear distance) on the floor for your test.

Now, block the path to the goal with first one, then later with multiple obstacles. Develop a strategy that will allow the vehicle to avoid the obstacles and then continue travelling to the specified goal along the shortest available path.

2a. Describe your strategy using a sketch.

2b. Add this function to your Blockly program from experimental task 1.

Tip: The Mecanum vehicle turns around the centre of the vehicle. All calculations to the path of travel, therefore, always relate to the centre point of the vehicle. If the travel should start and end on a line in front of the bumper, then the vehicle must finally be turned back in the direction of travel so that it will stop in front of the goal line.

Annex

# Mecanum vehicle with obstacle detection

## Required materials

PC for program development, local or via web interface.

USB cable or BLE or WiFi connection to transmit the program to the TXT4.0.

Course sheet with black line in a closed circle, 2 cm wide (from the Robotics TXT 4.0 Base Set).

## Further information

[1]	FRC Team 2605 (Bellingham, WA): How a Mecanum Drive Works. github.io

[2]	Online diagram editor for creating state diagrams (Format drawio): https://www.diagrammeditor.de/