AI sorting line

The classification line model is very different from the models we have worked with so far. It is not only much more complex in structure, but also in the task it fulfills. This model simulates how quality control and sorting would work in a factory. It would use a neural network to ensure that the different parts have been made correctly and have no defects, and use that information to classify them in case there is more than one component type.

The task of the sorting line is to take a photo of the different parts, sort them and send them to the right position or order them. 

However, if the neural network previously classified images, it now goes one step further. This network not only classifies images, but goes one step further. This mesh looks for objects in the image, shows us where those objects are, and then classifies them. With this information, our program decides which drawers the part should be sent to.

The foundation

If you're operating the sorting line, make sure that the lighting conditions are good and even so that the camera can see the parts clearly.

Turn on the AI 4.0 sorting track, then start the TXT 4.0 controller by long-pressing the ON/OFF button. As soon as the driver is started, the Add_On_Sortierstrecke_AI program [1] must first be loaded into the driver. You can find the program in Robo Pro Coding at:

Project -> New Example -> ->STEM Add-on AI -> Add_On_AI

If no program loads, select the program Add_On_AI under "File" and download it via "Upload".

Exercise 1:

In this case, we have the program and the neural network that have already been trained. The driver already has a trained mesh that uses the program you were running. Try some parts and see which ones are considered valid and which are not.

With the results you receive, can you tell us which parts are correct and which are defective?

Exercise 2:

One of the problems to avoid with neural networks is what is known as distortion. Let's summarize: Neural networks are trained by examples, just as we learn. In this case, you've been learning in the world for a few years, but the neural network has to learn in minutes, hours, or maybe days, like ChatGPT, for example. And as we already know, he only learns from examples. If you had seen all the tables with four legs and all the chairs with only one leg since you were born, and suddenly saw a table with only one leg in the middle, would you know that it was a table, or would you think that it was a chair?

The neural network would feel just like you, but it's even dumber. It is not enough to see an example and explain to him with just a photo that this table with one leg is a table. He has to see it many times, as many times as he has seen tables with four legs. That's why, from the beginning, we asked you to take the same number of photos of each object to do the training you've done so far.

Distortion occurs when we train neural networks with many more examples of one type than others. When we do this, the network always responds with what it has seen the most, and this is one of the most important problems to avoid. We need to train networks as evenly as possible and with the best possible data quality. But we'll talk about that later.

Optional_1: Train your leaderboard.

For this task, we do another training. In fact, object detection would be ideal. But you need a lot of examples, and the training can easily take hours. That's why we don't do it. Instead, we only perform image classifications. One advantage of object recognition is that you know where an object is in the image. However, we do not need this information for our rating line.

Use the program "Add_On_AI_Sorting_Line_tag_image"  to take the necessary photos. You can set the location (route) or use the default route. Take photos of the objects you want to classify and label them.

Then you do the training with the programs you know. After that, all you have to do is test the trained neural network and see how it works. You can use the "Add_On_AI_Sorting_Line_custom_model"  sample program or customize the label image program to  use the trained network.

With the new program, you can test your neural network again and see the results.

Cultivation

AI of the sorting line

Learn more

[1]	IA Classification of Program Model Paths: Add_On_AI

[2]	Manuelles TXT 4.0 – SSH Configuration

[3]	Phyton Installer, https://www.python.org/