# Name: __________	Class: ___	Date: ________

# Solution sheet model 1

# Windscreen wipers - Tandem

## Construction task

- The rotation of one axle makes the other axle rotate in the same (!) direction. This also works when the Cardan joint is bent and there is an angle between the axles.

- The greater the bend angle, the more irregular the rotary transmission will be. It no longer works at all from a certain limit onwards.

## Topic task

- The correct setting is an eccentricity of a few millimetres. This can easily be determined by trial and error.

- The crank has to be rotated twice for the windscreen wipers to move exactly one cycle up and down. This is due to the step-down ratio Z10 to Z20 (in short: 10:20) in the gear drive: One crank revolution, in other words one revolution of the Z10, moves the Z20 by 10 teeth as well – in other words by half a revolution. To achieve one revolution of the Z20 and thus one wiping cycle, the Z10 and thus the crank have to be turned twice.

- As far as overlapping of the wipers is concerned:

- The wiper on the right from the gear side (“from inside” in the vehicle) must be at the bottom, the one on the left at the top.

- If the right-hand wiper were at the top, the left-hand wiper would no longer be able to be raised - it would be blocked by the right-hand one.

- The end of the right-hand wiper blade near the centre of rotation only moves up and down a little. The end of the left-hand wiper blade away from the centre of rotation, on the other hand, moves a long way (it describes a large circular path around the centre of rotation). The right-hand wiper blade does not permit this large motion. This is why the left-hand wiper blade has to be on top.

## Experimental task

- The direct drive of the Z20 by the crank-Z10 reverses the direction of rotation of the Z20 during cranking. However, the speed of rotation does not change - it remains at the step-down ratio 10:20.

- From a certain eccentricity, the possible swivel range of the cranks is no longer sufficient for the offset range of the eccentric. The model then blocks. The wiper blades can only be moved back by cranking in the other direction. This means, however, that the purpose of the eccentric - the conversion of an even rotary motion into a back-and-forth or swivelling motion - is no longer fulfilled.

- Manual swivelling of the wipers can complete the wiper motion, but the crank itself then also rotates backwards and forwards. The eccentric does not move beyond its extreme positions (the “dead centres”). In other words, this mechanism only works in one direction: Drive at the crank, output at the wipers.

# Attachments

## Windscreen wipers - Tandem

# Further information

[1]	Wikipedia: Spur gear.

[2]	Wikipedia: Eccentric.

[3]	Wikipedia: Mechanical linkage.

[4]	Wikipedia: Universal joint.