# Name: __________	Class: ___	Date: ________

# Solution sheet 11 - Differential gear and pawl - Game

## Construction task

The game in its initial position:

## Topic tasks

- The Z40 rotates about itself, but the angle of inclination of the vertical axis remains constant.

- The Z40 does not rotate about its axis, only the inclination changes.

## Experimental task

The system of this model has two degrees of freedom, these are the two angles or rotation of the outer Z10 axles. This means that both the rotation of the Z40 around its own axis as well as the inclination of this axis can be adjusted in any way at the same time. If the set-up were higher (which can easily be achieved using a few angle girders as a substructure), the Z40 would be able to be rotated completely around the transverse axis.

In technical terms, that is the same as in a differential gear except that there is another (large) gear wheel missing at the “cage” of the Z40 itself.

# Attachments

## 11 - Differential gear and pawl - Game

# Further information

[1]	Thomas Püttmann: Das Differentialgetriebe (The differential gear). ft:pedia 4/2014, p. 12–19.

[2]	Wikipedia: Differential gear.

[3]	Johannes Wiesinger: Ausgleichsgetriebe (Differential gears). At www.kfztech.de.