# Name: __________	Class: ___	Date: ________

# Solution sheet 12 - Vice

## Construction task

- The transverse lever as the last element attached:

- The reinforcement axle (drawn yellow) and spring cams for reinforcement:

- The links for reinforcement at the front:

- The transverse link above the worm bearing:

- The longitudinal connector above the vice:

- The disadvantage now is that we can no longer clamp a long larger workpiece in crosswise, such as a wide ruler.

## Topic task

- It allows the vertical fischertechnik axle to be clamped really be clamped and held in this vertical position, for example - even if we pull or push the workpiece in all directions during machining.

- In its central position, the lever has a radius of

- r=90mm2+15mm2=105mm2=50.25mm

- This means that one revolution has a circumference of

- U=2πr=105mm∙π

- The mechanical advantage is thus 105mm∙π1mm≈330-fold!

- Since the operating lever is fitted loosely, it can be extended completely in one direction, almost doubling the lever length - and thus almost doubling the mechanical advantage!

## Experimental task

- The I-strut forms a right-angled triangle with the angle girders or the space between them. Counted in 15-mm pitch units, it has a side length of 4 units at the bottom and a side length of 3 units upwards. According to Pythagoras, the length of the lateral strut is thus 32+42=9+16=25=5 units. This works out exactly.

- It can be proved that the square roots of integers are always either integer or irrational, but can never be represented exactly by the fraction of two integers. Combinations of numbers like 3, 4 and 5, where this works, are so-called Pythagorean triples. And 5 pitch units each 15 mm in length result in exactly 75 mm - which is why the I-strut 75 fits mathematically exactly.

- Since the two angle girders in the base are fitted to identical angular blocks 30°, the distance between their lowest holes is 45 mm - no change compared to the distance of the angular blocks on the building plate.

- The I-strut is fastened exactly 3 pitch units, or exactly 45 mm away from the lowest hole. The imaginary lower line (45 mm horizontal), the rear angle girder (45 mm of it) and the I-strut form at least an isosceles triangle (the two sides known so far both have the same length of 45 mm).

- Due to the angular blocks 30°, the opening angle between the imaginary lower line and the line of the angle girder is exactly 60°. Since the sum of the angles inside a triangle is always 180° and the two known sides have the same length (45 mm), the two other inner angles are also exactly 60° in size. In other words, what we have here is an equilateral triangle - where all three sides have the same length, namely 45 mm. The I-strut 45 has exactly the length mathematically required.

# Attachments

## Vice

# Further information

[1]	Wikipedia: Vice.

[2]	Wikipedia: Flight.

[3]	Wikipedia: Worm drive.

[4]	Wikipedia: Module (gear wheel).

[5]	Thomas Püttmann: Zahnräder und Übersetzungen (Teil 2) (Gear wheels and transmission ratios (Part 2)). ft:pedia 3/2011, p. 25–28.

[6]	Wikipedia: Pythagoras.

[7]	Wikipedia: Pythagorean theorem.

[8]	Duden Learnattack GmbH: Pythagoreische Zahlentripel (Pythagorean triple). At lernhelfer.de.

[9]	Wikipedia: Pythagorean triple.