# Name: __________	Class: ___	Date: ________

# Solution sheet Model 13 - Inclined plane with small force gauge

## Construction task

- Without force gauge and set-up supports:

- With force gauge and set-up 45°:

- With force gauge and set-up 30°:

- Why is the set-up support 45° an exact fit?

- From the centre of the joint at the base to the strut adapters for suspending the struts, the track is 18 pitch units (à 15 mm) long. The length of the set-up support 45° to be constructed can be considered as the hypotenuse (long side) of a right-angled isosceles triangle, one leg of which runs from the suspension point 9 units down to the left (in the illustration of the model) and with the second leg perpendicular to it 9 units down to the right. There, the second leg exactly meets the base of the set-up. According to Pythagoras, we thus need a length of

- L=92+92=9∙2

- units.

- The structural connecting strip 21.2 has been designed for one pitch unit:

- 15mm∙2=21.213mm

- The X-strut 84.8 has been designed for a diagonal of 4 pitch units:

- 4∙15mm∙2=84.852mm

- (The labelling on the X-strut 84.8 would actually be more accurate rounded to one decimal place if it were 84.9.)

- This means that with two X-struts 84.8 and one connecting strip 21.2 between, we get exactly the length we require - within the context of production precision - to set up the track at exactly 45°.

- Why is the set-up support 30° an exact fit?

- An equilateral triangle is given. All inner angles are 60°. Then the upper half of this triangle has a half angle of 30° and a length of the right half side of exactly half the side length of the initial triangle:

L

30°

60°

90°

L/2

L/2

L

- This means: For a 30° set-up angle we need a set-up support which has exactly half the length of the 18 pitch units. This is

- 182∙15mm=9∙15mm=135mm

- This works perfectly with two I-struts 60 (2   4 pitch units) and one structural connecting strip 15 in between (the missing 9th pitch unit). So the set-up support works exactly.

## Topic task

- Qualitative consideration:

- First of all, there must be enough force acting to overcome the static friction of the wheels and their bearings. The carriage only rolls once this is the case, then slowed by rolling friction (the wheels on the carriers) and kinetic friction (the wheels on their axles).

- The larger the set-up angle, the greater the acceleration, because the greater the proportion of the force acting downwards parallel to the track – the parallel component of the force of gravity. We can divide the weight force which acts on the carriage into two components which act parallel or at right angles to the track:

- The weight force is shown as a green arrow, the two force components which result in the same force in (vector) sum are shown in red. With the set-up angle 0, there is thus no force acting parallel to the track, whereas when the track is vertical, the weight force is identical to the parallel component of the force of gravity and there is no force at right angles to the track.

- Qualitative consideration:

- The measuring results must be in the form of a sinus curve: The share of parallel component of the force of gravity in the weight force is sin(α), where α is the set-up angle.

- Half of the weight force as parallel component of the force of gravity is reached at 30° and not, as you might assume, at 45°. A purely geometric explanation is the division of the forces according to diagram 1. b) combined with the derivation that at 30° the leg of a right-angled triangle is exactly half as long as the hypotenuse (according to the solution of the topic task 2. b)). If you are already familiar with the sine function, it is also the result of sin30°=12.

## Experimental task

- The calculated value for the mass is arithmetically

- m=FgN=0.8N9.80665ms2=0.081578kg=81.578g

- But this is far too exact a figure - we cannot guarantee this accuracy! Since m and F are proportional to one another, the relative error is

- Erelative=0.02N0.8N=2.5%

- This means the measuring error in kg - assuming that we can really read off exactly to 0.02 N and without taking systematic errors into consideration is:

- ∆m=Reading errorMeasured value∙m=0.02N0.8N∙0.8N9.80665ms2=0.002039kg=2.0g

- At 30° set-up angle of the inclined plane, the parallel component of the force of gravity is half the weight force - in other words 0.4 N. The reading accuracy is still unchanged at 0.02 N. So the percentage error is now

- Erelative=0.02N0.4N=5.0%

- The error in determining the mass is now twice as high in percentage terms, because we are still measuring just as inaccurately in absolute terms as we do with the total force! The measuring error in kg is thus

- ∆m=Reading errorMeasured value∙m=0.02N0.4N∙0.8N9.80665ms2=0.004079kg=4.1g

- The determination of the mass of the carriage is thus much more accurate if we are clever about it - namely by doing it when the track is set up vertically. And even then it is not nearly as accurate as we know gravity acceleration to be.

Note (1): A measured result without specification of accuracy is not a reliable statement! How do you know that inaccuracy is not 30 %, 50 % or even several 100 %? It is only when we have information about the accuracy of the data that we know where we stand and can use it for sound further calculations.

Note (2): The accuracy of a value indicated by a measuring device is something quite different that the resolution of the display!

Example: A standard household body scale with a digital display usually has one decimal place in the kg figure, suggesting an accuracy of 0.1 kg or 100 g. Scales often even say "d = 100 g". But this d is only the resolution. It has nothing to do with the accuracy of the scale! The value shown can still be wrong by several kg! It is not for nothing that the manufacturers of such devices are typically wary of publishing information about the accuracy of their products. Now you know why.

Measuring instruments from which we require a certain known accuracy must therefore be calibrated. This is done by adjusting them in a suitable way so that they deliver the correct values for reference conditions that are known with sufficient accuracy. If this adjustment (which is a purely technical process) is carried out officially (which concerns legal issues), it is known as calibrating. Calibration is an adjustment carried out by an official body, a calibrated instrument is not just calibrated, it is officially calibrated. A device that has only been adjusted is not calibrated in the official sense.

# Attachments

## Inclined plane with small force gauge

# Further information

[1]	Wikipedia: Inclined plane.

[2]	Dennis Rudolph: Messfehler und Fehlerbetrachtung (Measuring errors and consideration of uncertainty). At gut-erklaert.de.

[3]	Ulf Konrad: Fehlerrechnung (Calculation of uncertainty). At ulfkonrad.de.

[4]	Ulf Konrad: Fehlerfortpflanzung (Propagation of uncertainty). At ulfkonrad.de.

[5]	Wikipedia: Propagation of uncertainty. Note: The maths used goes beyond secondary level.

[6]	Dr. Alexey Chizhik: Messfehler (Measuring errors). Georg-Augustin-Universität Göttingen. Note: This link is for those interested in seeing how far you can go with the calculation of uncertainty. The level is that of a physics degree. You can get to propagation of uncertainty by scrolling forward with the link on the right.