# Name: __________	Class: ___	Date: ________

# Solution sheet Model 2 - Windscreen wipers - Opposed

## Construction task

- The eccentricity must be just a few millimetres again. This is easy to adjust by trial and error.

## Topic task

- The purpose of the overlapping is to ensure that as small an area as possible remains unwiped in the centre of the windscreen. If the wipers did not overlap, a large area in the centre of the windscreen would be left which would be difficult to see through.

- The first asymmetry is created by the fact that the axis of rotation of the central eccentric is underneath the axes of rotation of the two cranks and not on the same plane. This means that the attachment point of the strut at the upper eccentric (which leads to the left-hand wiper crank) is nearer to the crank than the attachment point of the strut for the right-hand wiper crank is to this. The second asymmetry is the necessary different length of the two struts between central eccentric and wiper cranks.

- What is symmetrical, on the other hand, is the distance of the two strut attachment points from the axis of rotation. The two attachment points are the same distance away from this axis of rotation.

## Experimental task

- The positions of the different axles, eccentricities and lever lengths must be chosen suitably so that the required sequence of motion is produced. More difficult: A suitable combination of all these parameters must be found so that the mechanism does what it is supposed to do. With fischertechnik you can try it out for yourself quickly and easily by rebuilding the models. As an engineer, you would calculate the required parameters and maybe simulate them at the computer first - or even test them in detail on a model like the ones from fischertechnik.