# Name: __________	Class: ___	Date: ________

# Solution sheet 04 - Press - Eccentric press

## Construction task

## Topic task

- The gear drive is used to convert faster rotation at the crank into slower but more powerful rotation at the eccentric.

## Experimental task

- The press has the greatest pressing force in its bottom position (also known as “bottom dead centre”).

- In this position, one revolution of the crank makes the least difference in press position. The combination of eccentric and jointed lever acts like a step-down ratio here: Large travel by the eccentric (almost horizontal) only causes the press head to lower only a little and thus causes downward motion under the greatest possible force.

# Attachments

## Press - Eccentric press

# Further information

[1]	Wikipedia: Eccentric press.