# Name: __________	Class: ___	Date: ________

# Solution sheet Press - Toggle press

## Construction task

## Topic task

- The three components are:

- The gear wheel step-down gear,

- the eccentric mechanism and

- the toggle mechanism.

- Both the eccentric and the toggle versions have

- one fixed end (mounting of the crank or joint below the press end axle) and

- one moving end (end of the I-strut 30 on the toggle joint or press head).

Between these there is a joint (jointed mounting of the I-strut 30 by the crank or the middle joint building block in the toggle joint).

However, the drive differs: The crank is driven “from the start”, namely by rotating is mounted axle. But the toggle joint is driven by lateral pressure on the knee and not by rotating the upper joint building block, for example.

There is a “knee” in both cases, however! The eccentric drive is nothing other than a special form of toggle, but with the drive “from the start” rather than “at the knee”. This is why the result is the same at the end of the day: The greatest force (on the knee or on the workpiece) is built up at a dead centre.

Which means we are dealing with a “sequential switching” (“cascading”) of two toggle joints. This reinforces the effects of the individual mechanisms.

## Experimental task

- From the considerations of the topic task, it follows that the toggle press can build up significantly greater forces than the simple eccentric press. If the force becomes too great, however, the roof could “lift off” - after all, the “thigh” of the toggle mechanism is supported upwards on the roof.

# Attachments

## Press - Toggle press

# Further information

[1]	Wikipedia: Toggle.

[2]	Wikipedia: Toggle press.