# Name: __________	Class: ___	Date: ________

# Solution sheet Model 6 - Toggle joint with over-extension - Aircraft landing gear

## Construction task

## Topic task

- One force is the force through the weight of the aircraft, which the wheels have to bear. The second force pushes the wheels to the rear (seen in direction of travel) during landing, wheeling and particularly during braking (!).

- The girder (angle girder 60) which is vertical when extended, bears most of the weight force. However, the force to the rear cannot be absorbed by this, because the girder is mounted as a joint and could swing freely to the rear. So the force to the rear must be absorbed by the angular positioned lever with the toggle mechanism.

- If the toggle joint were not over-extended and thus locked due to the design, the wheels would simply be pushed to the rear. This would also remove the load-bearing effect of the vertical girder. The aircraft would crash land.

## Experimental task

- You would have to turn the crank by six revolutions for the Z30 to complete one revolution.

- 2010×3010=2×3=6.

- In other words, the step-down ratio of individual part-gears can be calculated using the ratio of the numbers of teeth of the gear wheels involved. The crank has to turn twice for the Z20 to revolve once. The Z10 attached to the same (!) axle as the Z20 has to turn three times for the Z30 to revolve once. If several such gears are mounted in succession, the total step-down ratio is the product of the individual step-down ratios. For this reason, the crank has to turn 2×3=6 times before the Z30 revolves once.