# Name: __________	Class: ___	Date: ________

# Model 7 - Counter

## Construction task

If digit stickers get lost, you can print them out yourself and glue them lightly to the Z20 using a twist-up adhesive stick. You only need a little adhesive; the sticker can then be removed again easily if necessary.

- If you turn the crank clockwise (to the right), the units Z20 revolves anti-clockwise (to the left), the tens Z20 to the right and the hundreds Z20 to the left again.

- The stickers must be attached to the Z20 in such a way that the counter counts upwards when the crank is turned clockwise:

0

9

8

7

6

4

3

2

1

5

0

1

2

3

4

6

7

8

9

5

0

9

8

7

6

4

3

2

1

5

Hundreds

Tens

Units

## Topic task

- Due to the step-down ratio of crank-Z10 to the first counting-Z20, the crank has to be turned twice for units position to complete one full revolution and the counter to count from 0 to 10.

- You have to turn the crank 2 times to get to the 10, so 20 times to get to 100 and 200 times to reach '1000', which corresponds to switching the counter from 999 to 000.

- You could extend further, identically built stages. But this does have two limits:

- There is play (inaccuracy) in the mechanisms. If there are two many counting places, they add up in such a way that the carry-over and the positioning of the counting-Z20s would no longer function reliably.

- Already at the hundreds carry-over (e.g. from 099 to 100), the force of the crank has to move three Z20s with the spring-loaded detents underneath. If there are too many stages, the force would no longer be sufficient to switch them all at the same time.

## Experimental task

- Quick adjustment can be made by setting the counter to the desired combination of digits by directly turning the Z20. To ensure that no carry-over moves the following Z20, it is best to proceed in reverse order, from the hundreds to the units place.

- The set-up is suitable for counting backwards to a limited extent (turning the crank anti-clockwise). However, the tabs must be adjusted very precisely for counting to work reliably in both directions.

# Attachments

## Counter

# Further information

[1]	Wikipedia: Abacus.

[2]	Wikipedia: Mechanical calculator.

[3]	David G. Hicks: Early Calculators. In The Museum of HP Calculators.

[4]	Rick Bensene: Bohn Contex Model 10 Mechanical Calculator. Example of a purely mechanical table-top calculator for the four basic arithmetic operations. In The Old Calculator Museum.

[5]	David G. Hicks: The Friden SRW. Mechanical calculator with square root function. In The Museum of HP Calculators.

[6]	Wikipedia: Slide rule.