# Name: __________	Class: ___	Date: ________

# Solution sheet 8 - Lift - Pawl

## Construction task

## Topic task

- Instructions for moving the lift up:

- Make sure that the pawl is resting properly on the crank gear wheel and can move easily.

- Crank clockwise until the required height is reached.

- The crank can now be released. The pawl secures the rope winch and the lift. The pawl must not be lifted without holding the crank tight!

- Instructions for moving the lift down:

- Hold the crank firmly!

- Then lift the pawl without releasing the crank! You might have to turn the crank clockwise a little to clear the pawl and allow it to move.

- To lower the lift, the crank must be turned slowly in an anti-clockwise direction. During this, the pawl must be raised.

- When the lift has reached its destination, the pawl must be lowered onto the crank gear wheel first, to secure this. After this, the crank may be released.

## Experimental task

- Possible weaknesses resulting in faulty function are at least:

- You can lift the pawl so far that it tips to the wrong side and becomes completely ineffective. This could be eliminated using a few more components attached to the carrier of the pawl, which prevent the pawl from being lifted too far.

- The class should try to find possible solutions, present and test them in groups!

- You could crank the gear wheel “upwards” so fast, that the pawl “flips” by itself due to the impact of the Z30 wheels and thus becomes ineffective again as in a). The solution is the same as for a), namely a top-mounted stop for the pawl.

- The clips on the pawl could be too tight, so that the pawl can no longer fall reliably onto the gear wheel by itself solely on account of its own weight. This can be achieved by setting the clips correctly - and in a real lift by lubricating all the joints.

- The pawl or the gear wheel could be worn and no longer engage properly. This can be checked during preventive maintenance and solved by replacing the pawl or the gear wheel in good time.

- The pawl or its mount could break due to material fatigue (or excess load). This can be prevented by correct design and regular testing (e.g. by the TÜV).

- When the lift is at the top, somebody could simply lift the pawl without holding the crank. This would be a classic operating error - a human error rather than a technical fault, and difficult to reliably avoid. This operating error cannot be avoided with the given design. However, training and regular instruction of the operating personnel can help.

# Attachments

## Link - Pawl

# Further information

[1]	Wikipedia: Ratchet.

[2]	Wikipedia: Rope winch.