# Name: __________	Class: ___	Date: ________

# Model 9 - Crane rope winch - Pulley (block and tackle)

## Construction task

fischertechnik may wish to generate and add individual pictures from its CAD system for the design tasks 1, 2, 3.

## Topic task

- About the drive lever:

- Since you can grab the lever at a greater distance from the axis of rotation than the crank, you need less force to pull or can lift heavier loads with the same amount of effort. On the other hand, a crank drive can be turned continually and potentially faster.

- You could lengthen the lever by adding further building blocks to increase leverage even further again.

- The price to pay for this is that you have to pull on the end of the lever for greater distances and lifting of the load may therefore be even slower.

- About the block and tackle:

- Every further deflection pulley reduces the force required for a given load. However, you also have to crank more (longer) on the rope drum to lift it just as far. This is a factor block and tackle: Every pulley changes the factor by which the necessary tensile force is reduced (the sources also show other variants of block and tackle).

- The jointed set-up is a simple way of ensuring that the force gauge automatically aligns itself to the direction of the acting force. If it were at an angle to this (not in line with the upper side of the rope pulley at the top of the crane), measurement would be falsified. This is easy to understand if you imagine the force gauge being perpendicular to the direction of the force - at this point at the latest it becomes clear that it won’t work like this. See the measurements on the inclined plane on task sheet 13 later as well.

## Experimental task

Within the context of (limited) measuring accuracy, the measurements should be an approximate indication of the theoretical factors.

# Attachments

## Crane rope winch - Pulley (block and tackle)

# Further information

[1]	Stefan Falk: Raffiniertes mit Achsen (Clever work with axles). ft:pedia 3/2013, p. 38–41.

[2]	Artur Fischer: Krane. fischertechnik hobby, Experimente und Modelle, hobby 2 Band 4 (Cranes. fischertechnik hobby, experiment and models, hobby 2 volume 4), Fischer-Werke, 1975.

[3]	Dirk Fox: Flaschenzug (Block and tackle). ft:pedia 2/2014, p. 4–10.

[4]	Wikipedia: Flaschenzug (Block and tackle).

[5]	Artur Fischer: Der Flaschenzug (The block and tackle). In: fischertechnik hobby, Experimente und Modelle, hobby 1 Band 1, (fischertechnik hobby, experiments and models, hobby 1 volume 1), Fischer-Werke, 1972, p. 40-43.

[6]	Wikipedia: Newton (unit).

[7]	Wikipedia: Spring constant.