# Name: __________	Class: ___	Date: ________

# Model 1 - Windscreen wipers - Tandem

## Construction task

Before you build the model, take one of the Cardan joints, insert two clip axles into both ends and test the function of the Cardan shaft by hand.

- Try out how the two axles rotate and the rotary motion - not only straight, but “bent” too – is transmitted from one axle to the other.

- Is it possible to “bend” the Cardan shaft as far as you want so that the rotary transmission still works?

- __________________________________________________

Now construct the model according to the building instructions.

## Topic task

Let us follow the flow of force from the crank to the “wiper blades”:

- From the crank, it goes via a gear drive onto the larger red gear wheel Z20 (which has 20 teeth, whereas the black Z10 has 10 teeth). Finally, transmission is from a Z10 to a Z20. The Z10 in the middle only ensures direction of rotation is reversed: Two neighbouring gear wheels rotate in opposite directions, the third thus rotates in the same direction as the first again.

- The main element is the eccentric. It is made up of a rotatable element (in this case the black plate with the axle which is inserted in the Z20) and a point located outside the centre of rotation (“ex-centric”), at which the motion - here through the I-strut 45 – is picked off. This mechanism converts the even rotary motion into a back-and-forth motion. Simple but effective!

- The back-and-forth motion is transmitted to a crank. So this rotates backwards and forwards as well when the crank is rotated.

- The motion is transmitted to a second crank via the horizontal I-strut 90. This moves – apart from the “play” in the mechanism - in exactly the same way as the first crank. This is a parallel four-bar linkage.

- The rotating swivel motion of the cranks is now to be transmitted to the windscreen wipers themselves. However, since these are attached at an angle (for a sloped windscreen), whereas our entire linkage is horizontal, we transmit the rotation “round the corner” via the Cardan joints.

- At the end of the day the windscreen wipers thus move in parallel. Both move up and down at the same time.

Now to the tasks:

- The eccentric must be adjusted. You have to set how far the attachment point of the I-strut 45 is away from the centre of rotation of the drive axle. The further away (the “more eccentric”) it is, the larger the swivel range of the windscreen wipers will be. Set the eccentric by moving the building block 7.5 in such a way that the wipers swivel straight right to the bottom but the mechanism does not block.

- How many revolutions are necessary at the crank until the windscreen wipers have completed exactly one cycle – up and down again? Why is this?

- __________________________________________________

- The two “wiper blades” (the two black I-struts 90) overlap when they are resting horizontally at the bottom.

- Which has to be lying on top, which underneath?

- __________________________________________________

- What happens if the wipers are lying on one another in the wrong order?

- __________________________________________________

__________________________________________________

__________________________________________________

- Describe in full sentences why this problem then occurs.

- __________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

## Experimental task

- Remove the middle gear wheel Z10. Then move the holder for the first Z10 so that it engages directly into the Z20 as shown here:

- Turn the crank. What has changed?

__________________________________________________

__________________________________________________

__________________________________________________

- Adjust the eccentric so that the attachment point of the I-strut 45 is an extremely long way away from the centre of rotation of the drive axis (“increase eccentricity”). What happens during cranking?

- __________________________________________________

__________________________________________________

__________________________________________________

- Try to “reverse” the drive: Rotate one of the wipers yourself. Swivel the wiper backwards and forwards. What do you observe?

- __________________________________________________

__________________________________________________

__________________________________________________

# Attachments

## Windscreen wipers - Tandem

# Further information

[1]	Wikipedia: Spur gear.

[2]	Wikipedia: Eccentric.

[3]	Wikipedia: Mechanical linkage.

[4]	Wikipedia: Universal joint.