# Name: __________	Class: ___	Date: ________

# Model 12 - Vice

## Construction task

A vice is used to clamp and hold a workpiece in place so that it can be worked (e.g. filed), keeping both your hands free. Vices are typically made of solid metal parts and are correspondingly heavy and sturdy. They are usually also fixed to a workbench or table.

- Construct the model according to the building instructions, but initially only up to and including the transversely mounted lever, which can be turned vigorously. Leave all the following reinforcements off initially. Clamp a pen or a fischertechnik axle in the vice in an upright vertical position. Turn the rotary lever clockwise until both “jaws” of the vice clamp the workpiece. Observe how the model buckles - we should do something to prevent this.

- As the next construction step, add the reinforcement: The metal axle, which is routed into the building blocks on the back of the vice, will protect the row of building blocks from buckling. To safely prevent the axle falling out, it is secured on both sides using one spring cam each.

- Now clamp the workpiece in place again. There will be much less buckling of the rear wall - but the worm bracket at the front will still buckle.

- In the next construction step, add the two red links 30.

- Try out the vice again. The two building blocks which are held together by the links no longer buckle as much. However, the worm bearing itself will still have to absorb large forces and buckle the front end.

- So in the next construction step we add the transverse link above the worm bearing.

- Now that's better, isn’t it? However, these are all still plastic parts, not solid iron components: The entire set-up can still be bent apart if enough clamping force is used.

- We can do something about that in the next step, by holding the front and rear together sturdily at the top as well, above the jaws.

- Now the structure is sturdy. However, this last reinforcement has also resulted in a disadvantage; perhaps it is a little exaggerated. Which disadvantage do we mean?

## Topic task

- What is the small slot left on purpose between the two “jaw chucks” on the back of the vice for (there is such a depression on real vices for the same reason)?

- __________________________________________________

__________________________________________________

__________________________________________________

- Let us calculate how much mechanical advantage the vice offers. How much force is needed to clamp the workpiece if we turn at the outside of the levers with a certain force? Here are a few parameters for this:

  - The worm “module” and its worm building block seat is its travel during one revolution. It is 1 mm for this black fischertechnik work.

  - The transverse strut is an I-strut 90. It is 90 mm long from end to end.

  - The two axle couplings are each inserted half-way on this. They are both 15 mm long.

If the transversely clamped lever is centred, what is the ratio of the path described by the outer end of one half of the lever in one complete revolution to the stroke of the worm in one revolution? Except for the friction losses, this is also the factor by which the force of the clamping is greater than the force with which you pull on (one) half of the operating lever.

__________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

- In a real vice, the operating lever sits very loosely and moves easily in the axis of rotation of the worm. This makes it possible to do something to increase the clamping force even further. What?

- __________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

## Experimental task

Would you like two little puzzles to solve?

With fischertechnik, the I-struts are used for attachment in longitudinal direction on components such as structural girders. The pitch of fischertechnik elements is 15 mm. This is the distance between two holes in the structural girders, for example. I-struts are always a multiple of 15 mm long.

X-struts, on the other hand, are intended for attachment at a 45° angle. This is why they have the same odd dimensions, namely integer multiples of 15 mm∙2 (you remember the famous Pythagorean theorem of course?).

The post of the vice is made completely of I-struts 75 and I-struts 45 - but all at angles!

- Why does the I-strut 75 fit exactly onto the back of the post exactly as installed on the structural girders?

- __________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

- Why do the I-struts 45 attached at an angle at the sides fit exactly there?

- __________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

# Attachments

## 12 - Vice

# Further information

[1]	Wikipedia: Vice.

[2]	Wikipedia: Flight.

[3]	Wikipedia: Worm drive.

[4]	Wikipedia: Module (gear wheel).

[5]	Thomas Püttmann: Zahnräder und Übersetzungen (Teil 2) (Gear wheels and transmission ratios (Part 2)). ft:pedia 3/2011, p. 25–28.

[6]	Wikipedia: Pythagoras.

[7]	Wikipedia: Pythagorean theorem.

[8]	Duden Learnattack GmbH: Pythagoreische Zahlentripel (Pythagorean triple). At lernhelfer.de.

[9]	Wikipedia: Pythagorean triple.