# Name: __________	Class: ___	Date: ________

# Model 13 - Inclined plane with small force gauge

## Construction task

- Construct the module up to and including the carriage in accordance with the building instructions, but without the force gauge and the two set-up supports.

- Make sure that the carriage can roll smoothly. Its four wheels must run smoothly. The two horizontal wheels are to keep it straight on track.

- You can lift the inclined plane - the whole track, in fact - and see how strongly a carriage placed on the end of it accelerates. When the carriage reaches the bottom, it is supported by the somewhat springy strut.

Use this to work on topic task 1.

- Construct the force gauge and install it (fold the intercepting strut to the side for this). Also construct the two set-up supports. This way we can also record and evaluate the forces that occur numerically. Make sure that the force gauge shows “0” on its scale when the spring is completely relaxed.

- Using your knowledge of the system behind the I- and X-struts from task sheet 12: Try to deduce why the “set-up support 45°“, which is made up of the “odd” X-strut lengths 84.8 mm and structural connecting strips 21.2, actually results in an erection angle of exactly 45° when it is correctly attached and aligned exactly vertically.

- __________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

- Why does the “set-up support 30°” with its I-struts 60 and structural connecting strips 15 result in an erection angle of exactly 30° when it is correctly attached and aligned exactly vertically?

- __________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

Use this to work on topic task 2.

## Topic task

- Put the track down on the table. If the table is straight, the track will now be exactly horizontal. For the following considerations, we call this state the set-up angle of 0° with respect to the horizontal.

- Slide the carriage to the end of the track (away from the jointed suspension on the base plate). Slowly lift the track by hand. Why does the carriage not start moving immediately there is any deflection at all in the plane?

- __________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

- Observe how the carriage accelerates more strongly if you release it with the track raised quite a way up. (The structural strut at the bottom intercepts it softly, but don’t overdo it - the model or components can otherwise become damaged.)

- Let us measure the force in Newtons (N) that the carriage exerts on the force gauge at different set-up angles:

  - With the track horizontal, the force will be 0 N.

  - If the track is set up vertically, it is the entire weight force of the carriage (you will have to hold it a little to stop it falling out of the track - without exerting any upward or downward force if possible).

  - But we are also interested in angles somewhere between 0° and 90°.

We have at least three possibilities to determine the set-up angle:

  - The two building blocks 7.5 on the bottom of the building plate are meant for placing a set square on them and holding it against the track. The zero point of the set square should be exactly in the centre of the hole of the jointed blocks. Then you can easily read off the current set-up angle at the thin slot in the link 45 which is inserted into the bottom of the track. You can do this with a partner - one of you holds the track, the other can read off the angle.

  - The two set-up supports 30° (the shorter one) and 45° (the longer one) are clipped into the strut adapters (which look like spring cams but have a round pin), which are about 2/3 along the length of the track. The given set-up angle is then reached when the struts are facing exactly vertically down and the building blocks 15 at the lower end are resting fully and straight on the base plate. Important: The building blocks are then not positioned in such a way that they should be slid into the grooves of the building plate - in this case the struts would not be exactly vertical. The building blocks simply rest flat on the building plate. The important point is that the struts run exactly vertically down from their suspension on the track.

- Enter the force measured in a table like the one below:

| - Set-up angle | - Force measured (in N) | - Proportion of the measured force to the weight force (the one at 90° set-up angle) in percent |
| --- | --- | --- |
| - 0° | - 0.0 N | - 0 % |
| - 15° |  |  |
| - 30° |  |  |
| - 45° |  |  |
| - 60° |  |  |
| - 75° |  |  |
| - 90° |  | - 100 % |

- At which angle is the force exerted just half as big (50 %) as the weight force?

- __________________________________________________

__________________________________________________

__________________________________________________

## Experimental task

We can also consider our inclined plane as a balance for the carriage which we are operating at different set-up angles. If we measure the complete weight force (in N) at 90° set-up angle, we can use gravity acceleration g (the acceleration experienced by a free-falling body on the earth’s surface due to gravity) to calculate the mass (in kg) of the carriage. The following formula applies:

F=m∙a

The force F is equal to the mass m of the body multiplied by acceleration a. Thus:

m=Fa

In our case, acceleration a is gravity acceleration. Average gravity acceleration (it is not exactly equal everywhere in the world due to not completely even distribution of mass) is gN=9.80665ms2. If we read off the force F in N (Newton) on the force gauge, we can also determine its mass as

m=FgN

We have a value for gN down to five decimal places - which is great! Does that mean we can calculate the mass m of our carriage in kg exactly down to five decimal places as well? To tenths or even hundredths of a gram?

This depends on how exactly we can measure the force at all!

With the track vertical (90° set-up angle), a weight force of 0.8 N is indicated, for example. Various different kinds of error possibilities come into play here:

- Systematic errors are caused by the fact that we are doing something fundamentally wrong. That sounds worse than it is: A lot of systematic errors cannot be avoided.

  - For example, we have an unknown amount of friction in the wheels on the carriage and in the bearing of the axle with the spring.

  - We might not have positioned the scale exactly in the right position - e.g. somewhat offset from the right position - and thus misread all force measurements by a certain amount.

  - The scale might not have been printed quite correctly.

  - We do not know what the gravity acceleration is exactly at the place where we are taking the  measurements (although it will be about 9.81ms2 everywhere).

  - We could be looking at the scale at an angle and thus cause a distorted reading.

  - Etc., etc. etc... It’s good to know systematic errors, but putting a number on them is usually difficult.

- Reading errors and statistical errors:

- We can “quantify” these, i.e. put them into numbers that we can use for calculations. We have to estimate how accurately we can read the scale. One graduation mark on the scale corresponds to 0.1 N. fischertechnik: If the final scale is printed differently, this part may have to be adapted to that. We can look carefully straight at it, adjust everything well and yet still read off at best a 1/10 scale division reliably and reproducibly. This would be 1/10 of 0.2 N = 0.02 N. So we cannot read the force more accurately than to 0.02 N!

But now we can calculate how accurate our measurement of force is in terms of percent: 0.02 N reading error with a weight of 0.8 N makes

0.02N0.8N=0.025=2.5%

In other words, we have a 2.5 % measuring error (systematic errors not included!).

With this accuracy value, we look at our equation again

m=FgN

Since m is proportional to F (gN is a constant), the mass calculated m will also have to be provided with an inaccuracy of 2.5 %.

- Use F=0.8 N, reading error 0.02 N and gN to calculate the mass of the carriage in kg or g and state the error to be expected in kg as well.

- __________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

- Repeat the measurement at the set-up angle at which the force on the force gauge is only half the magnitude of the weight force. Calculate, again using reading error 0.02 N and gN, the mass and the accuracy of calculation (again both in kg or g).

- __________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

# Attachments

## 13 - Inclined plane with small force gauge

# Further information

[1]	Wikipedia: Inclined plane.

[2]	Dennis Rudolph: Messfehler und Fehlerbetrachtung (Measuring errors and consideration of uncertainty). At gut-erklaert.de.

[3]	Ulf Konrad: Fehlerrechnung (Calculation of uncertainty). At ulfkonrad.de.

[4]	Ulf Konrad: Fehlerfortpflanzung (Propagation of uncertainty). At ulfkonrad.de.

[5]	Wikipedia: Propagation of uncertainty. Note: The maths used goes beyond secondary level.

[6]	Dr. Alexey Chizhik: Messfehler (Measuring errors). Georg-Augustin-Universität Göttingen. Note: This link is for those interested in seeing how far you can go with the calculation of uncertainty. The level is that of a physics degree. You can get to propagation of uncertainty by scrolling forward with the link on the right.