# Name: __________	Class: ___	Date: ________

# Model 2 - Windscreen wipers - Opposed

## Construction task

Construct the model according to the building instructions. Make sure the layout and alignment of the cranks and the eccentric struts is correct.

- Task: Again, adjust the eccentric in such a way that swivel range of the wipers is suitable. The wipers should lie flat at the bottom, but not block.

## Topic task

The two wiper blades (the black I-struts 90) overlap again.

- Why is this overlapping important in a real vehicle?

- __________________________________________________

__________________________________________________

__________________________________________________

- Because the wipers overlap, one is above the other again. This means the upper one should start moving upwards earlier than the lower one, so that the wiper carriers do not get in each other’s way or even damage each other. Although both wipers are moved by the same drive, they have to be controlled deviating slightly from one another. In other words, the drive is not symmetrical, it is asymmetrical. Find and describe two design features in the model that create this required asymmetry.

- __________________________________________________

__________________________________________________

__________________________________________________

## Experimental task

- Under the two red building blocks 15 with bore of the central eccentric in the middle, there are two building blocks 5 each so that the correct height is reached. You have four building blocks 5 left in the kit that you have not built into the model yet.

- Increase the position of the central eccentric axle by building one or two further building blocks 5 (green in the diagram) under the two building blocks 15 with bore. Observe how the mechanism does not work as well when the axis of rotation of the middle eccentric is no longer in a suitable position.

- __________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________